# NEBULA REPORT

Team ANULOG

Dwiti Suchak

Vyom

Abhay

# Table of Contents

This field updates automatically in Word and Google Docs. In Word, right-click inside it and choose Update Field (or press F9); in Google Docs, click the table and use the refresh icon.

Right-click and choose Update Field to build the table of contents.

# 1. Executive Summary

## 1.1 Project scope and demonstrated capability

Nebula is a Stage-1 automated, simulation-in-the-loop analog receiver

design-space exploration framework. It integrates parameterized CTLE design,

real-SPICE evaluation, S-parameter channel modelling, behavioral one-tap DFE

analysis, PPO and baseline optimizers, staged metric evaluation, caching,

provenance, checkpoints, and reporting. The original engineering problem is

to explore interacting receiver parameters without treating a nominal

simulation or an optimizer score as proof of a complete physical receiver.

The implemented circuit is a one-stage, source-degenerated SKY130 NMOS CTLE.

Its output is evaluated through a behavioral one-tap DFE so that

receiver-level eye metrics can be measured after a four-port channel model.

The project combines this electrical model with an AutoCkt-style PPO search

agent, Random Search, and the Cross-Entropy Method (CEM).

[VERIFIED FROM SOURCE CODE] The primary implementation is distributed across

`circuits/`, `simulator/`, `rl/`, `experiments/`, `analysis/`, and `tests/`.

The principal orchestration entry point is

`experiments/run_autockt_pipeline.py`; the core evaluation function is

`simulator.receiver.evaluate_receiver()`.

The system's design objective is not simply to produce a high reward. A

candidate must survive a sequence of electrical and receiver-level checks,

and the final report distinguishes PASS, FAIL, and NOT CLAIMED. This last

status is important: an unmeasured metric is not converted into a passing

metric by a default value.

## 1.2 Evidence summary

| Metric | Value | Unit | Source | Verification Status | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Current regression suite | 632 passed, 2 skipped | tests | Live `pytest` run | [VERIFIED FROM LOGS] | All executed repository tests passed; two were skipped. |
| v3 training updates | 100 | updates | `results/v3_tt_1000_summary.json` | [VERIFIED FROM RAW SIMULATION DATA] | A completed real-backend training artifact exists. |
| v3 training evaluations | 692 | evaluations | Same summary | [VERIFIED FROM RAW SIMULATION DATA] | Real-SPICE evaluation count recorded by the artifact. |
| Historical qualification | 36/36 | conditions | `docs/V3_QUALIFICATION_RESULTS_20260913.md` | [VERIFIED FROM LOGS] | Candidate-fidelity TT/SS/FF qualification for `pipeline_ep0`. |
| Fair PPO comparison | 0/20 | successes | `results/benchmark_report.json` | [VERIFIED FROM LOGS] | One-seed, budget-matched result; not evidence of general PPO superiority. |
| Whole-circuit area | Not available | mm² | `analysis/final_specification.py` | [NOT AVAILABLE] | Only partial MOS channel area is computed. |

The following nominal values are reported because they are present in the

fresh real-SPICE evidence package, not because they are representative of

every candidate or every operating condition:

| Metric | Value | Unit | Source | Verification Status | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Eye height | 1.55 | V | `results/llm_wrapper_runs/smoke_evidence_demo.json` | [VERIFIED FROM RAW SIMULATION DATA] | Locked-phase receiver eye result for that run. |
| Eye width | 0.73 | UI | Same | [VERIFIED FROM RAW SIMULATION DATA] | Unit-interval width for the measured eye. |
| HD3 | -75.62 | dB | Same | [VERIFIED FROM RAW SIMULATION DATA] | Measured at the documented HD3 test condition. |
| Input-referred noise | 0.372 | mVrms | Same | [VERIFIED FROM RAW SIMULATION DATA] | Integrated noise over the implemented noise-analysis band. |
| Observed DFE errors | 0 | count | Same | [VERIFIED FROM RAW SIMULATION DATA] | Zero errors in the finite simulated pattern; not a BER guarantee. |

[ENGINEERING INTERPRETATION] These results demonstrate an integrated

evaluation workflow and successful candidate snapshots. They do not establish

silicon behavior, post-layout performance, PCI-SIG compliance, complete-chip

area, or reliable convergence from arbitrary starting points.

## 1.3 Principal limitations

The current work is explicitly Stage 1. The tail source remains ideal, the

DFE is behavioral, and the resistors and capacitors are represented by ideal

SPICE elements rather than extracted layout devices. The optimizer computes

partial matched-MOS channel area, but not passive area, routing, diffusion,

contacts, guard rings, sampler area, DFE area, or layout overhead.

[REPORTED IN PPT] The presentation states the same limitations, including

behavioral DFE modeling, partial area estimation, incomplete SF/FS coverage,

and lack of continuous tuning proof. [VERIFIED FROM SOURCE CODE] The

repository documentation makes these boundaries more precise and records a

separate 60-condition grid that includes SF and FS. Consequently, the report

uses 36/36 only for the documented TT/SS/FF qualification grid and does not

describe it as full five-corner qualification.

## 1.4 Requirements-compliance matrix

The matrix below separates implementation from qualification. “Demonstrated”

means the repository contains direct source, test, or artifact evidence for

the stated scope; it does not imply silicon or protocol compliance.

| Original requirement | Target/specification | Current implementation or evidence | Status | Limitation or qualification |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 5-GT/s / 2.5-GHz Nyquist context | 5e9 bit/s NRZ; f_N=2.5 GHz | `simulator/stimulus.py` | Demonstrated | Data-rate context only; not PCI-SIG electrical or protocol compliance. |  |  |
| CTLE peaking | 3–12 dB in intended band | AC metric and hard gate in `simulator/receiver.py` | Demonstrated for evaluated candidates | Continuous tuning across the full band is not established. |  |  |
| One-stage CTLE | Parameterized source-degenerated CTLE | `circuits/blocks/ctle.spice` | Demonstrated | Stage-1 topology with ideal tail source. |  |  |
| Source degeneration | `RDEG |  | CDEG` branches | CTLE netlist and AC bench | Demonstrated | Passive devices are ideal SPICE elements, not extracted layout devices. |
| One-tap DFE | Previous-decision correction in receiver evaluation | `simulator/receiver_metrics.py` | Implemented but not fully qualified | Behavioral model; no physical sampler/DFE signoff. |  |  |
| Eye width | Implemented threshold above 0.4 UI at candidate fidelity | `dfe_eye_width_ui` metric | Demonstrated for selected nominal runs | Finite pattern and behavioral DFE scope. |  |  |
| Eye height | Implemented threshold above 0.1 V | `dfe_locked_phase_eye_height_v` | Demonstrated for selected nominal runs | Repository convention; original external requirement needs confirmation. |  |  |
| HD3 | Below -30 dB at 100 mVpp differential | `ctle_hd3.cir`, `hd3_db()` | Demonstrated for selected runs | Applies only to the documented 100 MHz test and amplitude. |  |  |
| Input-referred noise | Below 1.5 mVrms over implemented band | `ctle_noise.cir` and integration | Demonstrated for selected runs | Applies only to the documented 10 MHz–5 GHz analysis. |  |  |
| Power | CTLE power between 0 and 15 mW | DC/transient power metrics | Demonstrated for CTLE-level accounting | Total receiver power is unavailable. |  |  |
| Area | 0.05 mm² total-area target | Partial matched-MOS channel-area estimate | Partially demonstrated | Passives, routing, contacts, guard rings, sampler, DFE, and overhead are excluded. |  |  |
| TT/SS/FF/SF/FS | Five process corners | Full 60-point grid is defined | Implemented but not fully qualified | SF/FS completion is unavailable. |  |  |
| Supply variation | 1.71, 1.80, 1.89 V | `PVT_GRID` and qualification grid | Demonstrated for historical TT/SS/FF scope | Not evidence of full five-corner qualification. |  |  |
| Temperature range | 0, 27, 75, 125 °C | `PVT_GRID` | Demonstrated for historical grid scope | Fresh final-fidelity completion is unavailable. |  |  |
| Near-optimality | Prove candidate is near the best feasible design | PPO/RS/CEM comparisons | Not established | No exhaustive optimum, bound, or statistically powered ranking exists. |  |  |
| Runtime reduction | Demonstrate lower cost than exhaustive search | Caching, staging, early rejection, measured runtimes | Partially demonstrated | No controlled end-to-end reduction factor versus an exhaustive baseline. |  |  |
| Human intervention | Eliminate manual engineering decisions | UI, wrapper, checkpoints, reports | Not established | Targets, channel, model, validation scope, and interpretation still require engineering choices. |  |  |

# 2. Project Objective and Design Requirements

## 2.1 Design objective

The objective is to automate the search for a circuit parameter vector

that satisfies receiver-level and circuit-level constraints under a selected

simulation condition. Here, R_LOAD and R_DEG are resistances in ohms,

C_DEG is capacitance in farads, I_TAIL is tail current in amperes,

V_DFE is the behavioral tap voltage in volts, W_MOS and L_MOS are

transistor dimensions in micrometers, and M_MOS is an integer multiplier.

The first two PPO generations expose only the first five entries; v3 exposes

all eight.

[VERIFIED FROM SOURCE CODE] The target specification is represented by four

primary quantities: minimum eye height, minimum eye width, minimum decision

margin, and maximum CTLE power. Additional hard checks include AC peaking,

HD3, noise, and zero observed DFE decision errors when those stages are run.

The optimization problem is therefore constrained rather than a free-form

regression problem. A candidate is acceptable only if the relevant metrics

m_i(θ, c) satisfy their direction-specific limits under condition c:

- m_eye,height(θ, c) ≥ T_eye,height

- m_eye,width(θ, c) ≥ T_eye,width

- m_margin(θ, c) ≥ T_margin

- m_power(θ, c) ≤ T_power

The actual threshold values are read from the target and project contracts;

they must not be inferred from a plot or from a presentation screenshot.

Where a run did not measure a metric, the report status is NOT CLAIMED.

## 2.2 Receiver-level requirements

| Requirement | Meaning | Evidence status |
| --- | --- | --- |
| Eye height | Vertical opening of the sampled receiver eye, in volts | [VERIFIED FROM SOURCE CODE] Metric is computed by `receiver_metrics.py`. |
| Eye width | Horizontal opening normalized to one unit interval | [VERIFIED FROM SOURCE CODE] Metric is computed from sampled phases. |
| Decision margin | Minimum separation used by the DFE decision process | [VERIFIED FROM SOURCE CODE] Metric is part of the receiver result contract. |
| CTLE power | Reported CTLE supply power, in watts | [VERIFIED FROM SOURCE CODE] DC stage computes and gates it. |
| HD3 | Third-harmonic distortion relative to the fundamental, in dB | [VERIFIED FROM SOURCE CODE] Measured by the HD3 bench when enabled. |
| Input-referred noise | Integrated input-referred noise, in Vrms | [VERIFIED FROM SOURCE CODE] Measured by the noise bench when enabled. |
| PVT robustness | Repeated qualification over defined corner, voltage, and temperature conditions | [VERIFIED FROM LOGS] Demonstrated for the documented 36-point grid in one historical result. |
| Total area | Complete physical receiver area | [NOT AVAILABLE] Only partial MOS channel area exists. |

## 2.3 Requirements that remain conditional or unavailable

[ENGINEERING INTERPRETATION] Eye metrics measured after a behavioral DFE are

useful for evaluating the algorithmic receiver contract, but they do not by

themselves demonstrate that a physical sampler, clocking circuit, or DFE can

implement the same correction. Similarly, a CTLE power value is not total

receiver power.

The supplied evidence does not contain a post-layout extraction, a DRC-clean

layout, silicon measurements, or commercial-SPICE cross-validation. It also

does not establish continuous compliance over every frequency in the stated

tuning interval. These items are therefore marked [NOT AVAILABLE] rather than

treated as implied by successful nominal simulations.

## 2.4 Figure and table plan for this chapter

Recommended figures are a requirements-to-metrics traceability diagram and a

constraint-direction diagram showing lower-bound and upper-bound metrics. The

figure should be generated from `rl/target_spec.py`,

`simulator/receiver.py`, and `analysis/final_specification.py`; no threshold

should be added from the PPT unless it is also present in source code.

# 3. Evidence Classification

## 3.1 Evidence hierarchy

This report uses a deliberately conservative hierarchy. Source code defines

what the system is intended to do; raw simulation artifacts show what a

particular run produced; logs establish execution and timing; documentation

explains historical decisions; and the PPT supplies presentation context.

These sources are complementary but are not interchangeable.

| Label | Use in this report | Example |
| --- | --- | --- |
| [VERIFIED FROM SOURCE CODE] | Directly confirmed from implementation or tests | PPO action deltas, SPICE stage order, PVT grid construction |
| [VERIFIED FROM RAW SIMULATION DATA] | Read from JSON/JSONL or waveform-derived result artifacts | Eye height, HD3, noise, candidate pass status |
| [VERIFIED FROM LOGS] | Confirmed by command output or run logs | Live test count, runtime, completed evaluations |
| [REPORTED IN PPT] | Present in the uploaded slide deck but not independently sufficient | Dashboard screenshots or slide-level summary claims |
| [ENGINEERING INTERPRETATION] | Reasoned explanation of design significance | Why staged rejection reduces expected runtime |
| [NOT AVAILABLE] | No traceable evidence exists in the supplied materials | Total receiver area, silicon validation |
| [REQUIRES FURTHER VERIFICATION] | Evidence exists but is incomplete, historical, or not independently reproduced | Historical qualification at a different fidelity |

## 3.2 Fresh, historical, synthetic, and planned evidence

Results are separated into four categories:

1. Fresh real-SPICE evidence is identified by its raw result artifact or

execution log and includes the current test run and wrapper demonstrations.

2. Historical real-SPICE evidence exists in committed result artifacts

and documentation but was not independently rerun for this report. The

36/36 qualification result is in this category.

3. Synthetic evidence exercises software mechanics without ngspice. It

can show that PPO learns a controlled algebraic landscape, but it cannot

prove circuit performance.

4. Planned or missing evidence identifies work described in plans or

limitations but not supported by a completed artifact.

[VERIFIED FROM SOURCE CODE] The synthetic benchmark is explicitly labelled as

non-circuit evidence in both its implementation and its tests. This prevents

a fast synthetic training curve from being used as a substitute for a real

SPICE result.

## 3.3 Reproducibility procedure

A reproducible claim in this report must identify, where available:

- repository revision and branch;

- source netlists and model-library identity;

- channel file, port map, and checksum;

- PPO version, checkpoint metadata, exact parameter grids, and seed;

- simulation fidelity and operating condition;

- target specification and metric thresholds;

- result file or live command that produced the value;

- whether the result was fresh, historical, or synthetic.

[VERIFIED FROM SOURCE CODE] `simulator/provenance.py` hashes implementation

and model inputs, and `simulator/ngspice.py` records executable and solver

identity. [VERIFIED FROM SOURCE CODE] v3 checkpoints store a metadata envelope

because the policy cannot safely be interpreted without its state schema and

parameter grid. [ENGINEERING INTERPRETATION] These mechanisms reduce accidental

mixing of incompatible results, although they do not replace independent

reruns.

## 3.4 Handling discrepancies

The repository contains several result families produced at different times,

fidelities, and interfaces. Values are not merged merely because their labels

look similar. For example, a nominal eye-width value from a fresh wrapper run

is not substituted into a historical PVT table, and a presentation screenshot

is not used to replace a raw JSON artifact.

The PPT was inspected slide by slide, including its embedded images. It is

used as [REPORTED IN PPT] context only. When a slide states “36 tested PVT

conditions,” this report cross-references `QUALIFICATION_PVT_GRID` and states

the precise TT/SS/FF scope. The separate SF/FS corners in the 60-condition

grid remain outside the demonstrated qualification claim.

## 3.5 Reproducibility table

| Evidence item | Repository source | Status | Reproduction note |
| --- | --- | --- | --- |
| Source revision | Git branch and HEAD | [VERIFIED FROM LOGS] | `rl-ppo-v2-study`, HEAD `11101a5`. |
| Unit/regression tests | `tests/` | [VERIFIED FROM LOGS] | `632 passed, 2 skipped, 27 subtests`. |
| v3 checkpoint | `results/ppo_v3_tt_1000_policy.pt` | [VERIFIED FROM RAW SIMULATION DATA] | Metadata-bearing v3 policy accepted by the v3 loader. |
| Historical PVT result | `results/v3_qualification_final_20260913.json` | [REQUIRES FURTHER VERIFICATION] | Present and documented, not rerun in this pass. |
| Synthetic training | `rl/synthetic_benchmark.py` and results | [VERIFIED FROM SOURCE CODE] | Software-mechanics evidence only. |
| Whole-chip physical area | No complete artifact | [NOT AVAILABLE] | Must not be reconstructed from partial channel area. |

# 4. System Architecture

## 4.1 Architectural layers

Nebula is organized as a layered pipeline rather than a single monolithic

optimization script:

1. Interface layer. The local web UI and the natural-language wrapper

accept targets, backend choices, checkpoint selections, channel options,

and validation settings.

2. Orchestration layer. `experiments/run_autockt_pipeline.py` validates

the target, generates candidates, filters nominal results, optionally runs

PVT and final measurements, and writes structured outputs.

3. Optimization layer. PPO, Random Search, and CEM produce parameter

candidates under their respective contracts.

4. Evaluation layer. `simulator.receiver.evaluate_receiver()` performs

staged DC, AC, transient, channel, noise, HD3, and receiver analyses.

5. Analysis layer. Specification assessment, PVT selection, comparison

reports, plots, schematic export, and dashboard data consume structured

results rather than re-running simulations.

[VERIFIED FROM SOURCE CODE] The web UI intentionally runs the pipeline in a

subprocess and does not import PPO, simulator, reward, or PVT-selection logic

into the server process. The LLM wrapper also constructs a pipeline command

and consumes its JSON result. This gives the interfaces a common source of

truth and provides process isolation for long-running or native-SPICE work.

## 4.2 End-to-end data flow

The principal data flow is:

TargetSpec → checkpoint / optimizer → θ → ReceiverEvaluation →

metric assessment → candidate selection → report and netlist.

The candidate vector θ is rendered into SPICE parameters. The

receiver evaluation returns stage results, measured metrics, violations,

warnings, failure stage, runtime, evaluation identity, and provenance. The

pipeline then treats the result as structured data: it does not infer a pass

from a missing value and it does not allow an incomplete PVT run to qualify a

design.

## 4.3 Failure-aware control flow

Each stage can return success or a structured failure. The receiver stops on a

hard failure, records the stage name, and avoids spending later-stage runtime

on a candidate that has already violated an earlier requirement. Typical

failure labels include `setup`, `dc`, `ac`, `ctle_transient`, `channel`,

`transient`, `noise`, `hd3`, and `internal` at the receiver level; the lower

ngspice contract additionally distinguishes launch, timeout, parsing,

measurement, and model-library failures.

[ENGINEERING INTERPRETATION] Early rejection is both an engineering safety

mechanism and a runtime strategy. It prevents a nominally invalid bias point

from being presented as a meaningful receiver result, while allowing the

optimizer to spend its limited evaluation budget on candidates that reach

informative metrics.

## 4.4 Architecture evidence table

| Layer | Main implementation | Input | Output | Evidence |
| --- | --- | --- | --- | --- |
| Web interface | `experiments/web_ui.py` | User configuration | Subprocess request and displayed artifacts | [VERIFIED FROM SOURCE CODE] |
| Natural-language wrapper | `nebula/llm_wrapper.py` | Text target/request | Validated pipeline arguments | [VERIFIED FROM SOURCE CODE] |
| Pipeline | `experiments/run_autockt_pipeline.py` | Target, checkpoint, backend, conditions | Candidate and qualification JSON | [VERIFIED FROM SOURCE CODE] |
| PPO environment | `rl/autockt_env.py` and `rl/trainer.py` | State, action, evaluator result | Transition and reward | [VERIFIED FROM SOURCE CODE] |
| Receiver evaluator | `simulator/receiver.py` | Parameters and simulation conditions | `ReceiverEvaluation` | [VERIFIED FROM SOURCE CODE] |
| ngspice adapter | `simulator/ngspice.py` | Rendered netlist and model library | Typed simulation result | [VERIFIED FROM SOURCE CODE] |
| Analysis/reporting | `analysis/` | Structured result artifacts | Verdicts, plots, summaries, exports | [VERIFIED FROM SOURCE CODE] |

## 4.5 Candidate selection and qualification boundary

Candidate generation and qualification are deliberately separate. PPO may

produce a candidate at a cheaper training fidelity, but the pipeline can

re-evaluate selected designs at candidate or final fidelity and then apply a

PVT condition set. A candidate that passes nominal metrics is therefore not

automatically PVT-qualified.

The output report contains PASS, FAIL, and NOT CLAIMED rows. A PVT row is

only promoted when the required condition set is actually evaluated and the

qualification logic accepts it. Incomplete or failed checks remain visible as

limitations rather than being silently omitted.

# 5. PCIe Gen-2 Receiver and Channel Model

## 5.1 Signaling context

[VERIFIED FROM SOURCE CODE] `simulator/stimulus.py` defines the receiver

stimulus bit rate as 5e9 bits/s. For binary NRZ signaling, the unit interval

is the time occupied by one bit:

where R_b is the bit rate in bits/s and UI is in seconds. The associated

Nyquist frequency is f_N=R_b/2. For the implemented 5 Gbit/s setting,

f_N is 2.5 GHz by this definition. This is the frequency region in which a

lossy channel and an equalizer must preserve enough signal separation for the

receiver's eye metrics to remain positive.

[ENGINEERING INTERPRETATION] The report uses “PCIe Gen-2-rate” to describe the

implemented 5 GT/s stimulus context, not to claim protocol compliance. A

complete PCIe receiver claim would also require protocol, package, clocking,

jitter, calibration, and compliance-test evidence. Such evidence is not

available in the provided evidence.

## 5.2 Differential signal path

The implemented signal path is:

v_TX,d(t) → H_CH,d(f) → v_IN,d(t) → CTLE → v_OUT,d(t) →

sampling and DFE → b_hat[k].

Here v_TX,d(t) is the transmitted differential waveform in volts,

H_CH,d(f) is the channel's differential transfer function,

v_IN,d(t) is the filtered receiver input, v_OUT,d(t) is the CTLE

differential output, and b_hat[k] is the binary decision for symbol k.

[VERIFIED FROM SOURCE CODE] `simulator.receiver._run_transient()` first creates

a deterministic NRZ stimulus, filters its differential waveform through the

loaded S4P channel, writes the resulting waveform into a temporary SPICE

stimulus include, and then evaluates `receiver_transient.cir`. The bench

includes 50-ohm source resistance per leg, a differential receiver

termination, optional input capacitance, and the CTLE load capacitance from

the simulation conditions.

## 5.3 Touchstone S-parameter representation

[VERIFIED FROM SOURCE CODE] `simulator.channel.load_s4p()` reads a Touchstone

1.x four-port file. Each frequency row contains the frequency followed by

the 16 complex S-parameters in full-matrix row-major order. The parser

supports RI, magnitude-angle, and dB-angle formats and requires a declared

reference impedance. The parsed matrix is represented as

where S_ij is the complex scattering response at frequency f from

source port j to destination port i. Frequencies must be strictly

increasing, and the parser rejects truncated or unsupported data.

The differential transfer is formed from the selected transmit and receive

ports as

The symbols t+ and t- identify the positive and negative transmitter

ports, while r+ and r- identify the corresponding receiver ports. The

factor 1/2 is part of the implemented mixed-mode combination.

[VERIFIED FROM SOURCE CODE] The port map is explicit and validated to use

ports 1, 2, 3, and 4 exactly once. The IEEE/IBM metadata selects physical

ports 1 and 3 for the transmitter pair and 2 and 4 for the receiver pair,

because its documented dominant forward paths are S21 and S43. The synthetic

fixture uses the repository's default mapping unless another map is supplied.

## 5.4 Channel loss and delay

The channel insertion loss reported by Nebula is

The implementation interpolates the complex transfer by interpolating

magnitude and unwrapped phase separately. [VERIFIED FROM SOURCE CODE]

`S4PChannel.insertion_loss_db()` applies this calculation at requested

frequencies, and the validation path evaluates 1.25 GHz, 2.5 GHz, and 5 GHz.

Bulk delay is estimated from a least-squares line fit to unwrapped phase:

where φ is phase in radians, f is frequency in hertz, a is the

phase slope in radians per hertz, and τ_bulk is delay in seconds.

[VERIFIED FROM SOURCE CODE] The fit uses the 100 MHz to 5 GHz interval and

rejects a material negative delay.

The integer delay alignment is

implemented with a round-half-up rule that stabilizes floating-point ties.

The remaining fractional phase is retained as the sampling reference phase.

## 5.5 Channel validation gates

[VERIFIED FROM SOURCE CODE] `validate_s4p_channel()` applies deterministic

preflight checks before receiver transient evaluation.

| Check | Implementation requirement | Unit | Evidence |
| --- | --- | --- | --- |
| Reference impedance | Exactly 50 within the implementation tolerance | ohm | [VERIFIED FROM SOURCE CODE] |
| Frequency coverage | Data extends through at least 5e9 | Hz | [VERIFIED FROM SOURCE CODE] |
| Finite transfer | All differential-transfer values finite | — | [VERIFIED FROM SOURCE CODE] |
| Passivity screen | Maximum singular value no greater than 1.01 | dimensionless | [VERIFIED FROM SOURCE CODE] |
| Delay | No material negative fitted bulk delay | s | [VERIFIED FROM SOURCE CODE] |
| Causality screen | Negative-time energy ratio no greater than 0.05 | dimensionless | [VERIFIED FROM SOURCE CODE] |

The passivity diagnostic computes the largest singular value of the full S4P

matrix over the validation band. The causality diagnostic removes the fitted

transport delay, performs a band-limited inverse real FFT, and compares the

energy in the wrapped negative-time half with total impulse-response energy.

[ENGINEERING INTERPRETATION] These are deterministic integration gates, not a

formal rational-model or VNA signoff. The source code explicitly describes

them as a qualification gate and recommends trusted external modelling tools

for final signoff.

## 5.6 Available channel evidence

| Channel | Role | Evidence status | Limitation |
| --- | --- | --- | --- |
| `synthetic_regression.s4p` | Deterministic software regression | [VERIFIED FROM DOCUMENTATION] | Synthetic and not circuit evidence. |
| `ieee802_ibm_20db_thru.s4p` | Public lossy reference channel | [VERIFIED FROM RAW SIMULATION DATA] | Its metadata states it is not PCIe compliance evidence. |

[REPORTED IN PPT] The presentation shows channel and receiver-flow material.

[VERIFIED FROM DOCUMENTATION] The repository sidecar is the controlling

source for channel identity, provenance, port assignment, and compliance

limitations.

# 6. CTLE Circuit Topology

## 6.1 Differential-pair structure

[VERIFIED FROM SOURCE CODE] `circuits/blocks/ctle.spice` defines the reusable

`CTLE` subcircuit. It contains two equal load resistors from `vdd` to `outp`

and `outn`, two matched `sky130_fd_pr__nfet_01v8` devices, two identical

source-degeneration branches, and an ideal DC tail-current source.

The differential input and output variables are

The reversed output order is deliberate. [VERIFIED FROM SOURCE CODE] The AC,

transient, and noise benches comment that the common-source pair inverts the

signal and therefore uses `outn-outp` as the logical receiver polarity.

## 6.2 Implemented circuit parameters

| Parameter | Meaning | Range/value | Source file | Engineering role |
| --- | --- | --- | --- | --- |
| `RLOAD` | Each drain load resistance | Baseline 1 kΩ; automated legacy range 100 Ω–10 kΩ | `README.md`, `simulator/design_schema.py` | Converts drain current variation into output voltage; trades gain against bandwidth and operating point. |
| `RDEG` | Each source-degeneration resistance | Baseline 1 kΩ; automated legacy range 10 Ω–10 kΩ | Same | Reduces local feedback-dependent gain and improves linearity; shapes the low-frequency degeneration. |
| `CDEG` | Capacitor parallel to each degeneration resistor | Baseline 0.5 pF; automated legacy range 10 fF–10 pF | Same | Reduces degeneration at higher frequency and creates the frequency-dependent equalization transition. |
| `ITAIL_VAL` | Ideal differential-pair tail current | Baseline 100 µA; automated range 10 µA–1 mA | Same | Sets bias current, transconductance, speed, and CTLE power. |
| `DFE_TAP` | Behavioral feedback tap voltage | -0.4 V to 0.4 V | `ReceiverParameters.validate()` | Compensates the previous-symbol contribution in the software DFE. |
| `MOS_W` | Matched NMOS width | v3 grid/bounds 0.42–100 µm | `simulator/design_schema.py` | Changes drive strength, capacitance, and partial channel area. |
| `MOS_L` | Matched NMOS channel length | v3 grid/bounds 0.15–1.0 µm | Same | Changes device electrical behavior and partial channel area. |
| `MOS_M` | Matched device multiplicity | Integer 1–16 | Same | Represents parallel device instances and affects drive and partial area. |

The baseline values above are [VERIFIED FROM DOCUMENTATION] values recorded

by the repository. The automated ranges and validation bounds are

[VERIFIED FROM SOURCE CODE]. A range is not a manufacturability claim: the

project documentation explicitly says the v3 grids are bounded sizing grids,

not proof of DRC closure.

## 6.3 Small-signal interpretation

For a symmetric differential pair, a simplified small-signal view gives an

output differential gain of the form

where g_m is effective transconductance in siemens, Z_load is the

frequency-dependent load impedance in ohms, and A_deg represents the

source-degeneration feedback factor. This equation is an engineering model,

not the exact SKY130 compact-model equation used by ngspice.

[ENGINEERING INTERPRETATION] Increasing `RLOAD` can increase low-frequency

voltage gain but also increases the output-node time constant with parasitic

and explicit load capacitance. Increasing `RDEG` strengthens local feedback,

which generally lowers gain and improves linearity. `CDEG` bypasses part of

that degeneration at higher frequency, allowing a controlled transition from

lower low-frequency gain to higher high-frequency gain. The exact result is

determined by the transistor model, bias point, loads, and channel, so these

directions should not be read as monotonic guarantees for every candidate.

## 6.4 Source degeneration and peaking

The two branches are implemented as

Here s=j2π f, f is frequency in hertz, R_DEG is ohms, and

C_DEG is farads. The characteristic transition frequency of this simple

parallel network is

[ENGINEERING INTERPRETATION] This equation explains why the R/C pair is a

natural CTLE control: below the transition, the resistor contributes strong

degeneration; above it, the capacitor lowers the effective degeneration

impedance. The actual response can include additional poles and zeros from

the MOS devices, loads, and wiring represented by the model.

The project does not use “peaking” as an informal visual judgement. It

extracts gain from the AC trace and computes the difference between the peak

gain in the intended 1.25–2.5 GHz band and the gain at 100 MHz:

[VERIFIED FROM SOURCE CODE] `simulator.waveform.ac_metrics()` also classifies

the response as a local peak, controlled shelf, or uncontrolled rise and

checks the out-of-band peak and shelf behavior.

## 6.5 Tail current and MOS sizing

[VERIFIED FROM SOURCE CODE] The CTLE subcircuit uses an ideal source

`ITAIL tail vss DC {ITAIL_VAL}`. The implementation does not contain a

transistor-level bias generator in this Stage-1 block.

[ENGINEERING INTERPRETATION] For a fixed topology, I_TAIL changes the

available branch current and therefore the MOS operating point and effective

g_m. Higher current can improve speed or linearity but increases static

power. The actual trade-off is evaluated by ngspice rather than assumed from

a hand-derived square-law model.

The matched sizing group is represented by one width, one length, and one

multiplier shared by both devices. The partial channel-area expression used

by v3 is

where the factor 2 represents the two matched transistors and the dimensions

are expressed consistently in the area-reporting calculation. [VERIFIED FROM

SOURCE CODE] This quantity is explicitly partial and is never promoted to a

total-area PASS.

## 6.6 Circuit and simulation trade-offs

| Design change | Intended benefit | Possible cost | Evidence boundary |
| --- | --- | --- | --- |
| Increase `RLOAD` | More voltage conversion from drain current | Lower bandwidth or altered output headroom | [ENGINEERING INTERPRETATION] Exact behavior is simulator-dependent. |
| Increase `RDEG` | More degeneration and potentially improved linearity | Lower gain and altered peaking | [ENGINEERING INTERPRETATION] |
| Increase `CDEG` | Earlier high-frequency degeneration bypass | Excessive peaking or noise shaping | [ENGINEERING INTERPRETATION] |
| Increase `ITAIL_VAL` | More bias current and possible speed/linearity improvement | More CTLE power | [ENGINEERING INTERPRETATION] |
| Increase `MOS_W` or `MOS_M` | More drive and transconductance opportunity | More capacitance and partial area | [ENGINEERING INTERPRETATION] |
| Increase `MOS_L` | Changes current and intrinsic device behavior | Bandwidth and area trade-offs | [ENGINEERING INTERPRETATION] |

The table describes engineering rationale, not guaranteed one-dimensional

monotonicity. [REQUIRES FURTHER VERIFICATION] A complete sensitivity study

with matched real-SPICE perturbations for every geometry and passive variable

is not available in the provided evidence.

Figure 1. Source-degenerated differential CTLE schematic supplied in the

Nebula presentation. It shows the matched MOS pair, resistive loads, parallel

degeneration R/C branches, and ideal tail-current source. Evidence:

`report_generation/figures/ppt_ctle_topology.png`, extracted from the uploaded

PPT; [REPORTED IN PPT]. The netlist in `circuits/blocks/ctle.spice` remains

the primary implementation evidence.

# 7. Behavioral One-Tap DFE

## 7.1 Purpose and model boundary

[VERIFIED FROM SOURCE CODE] `simulator/receiver_metrics.py` implements a

stateful one-tap DFE. The DFE operates on sampled CTLE output values after

the channel and does not instantiate a transistor-level sampler, latch,

feedback amplifier, or physical decision circuit.

The DFE's purpose is to remove an estimated contribution from the preceding

symbol. Let x[k] be the raw sampled voltage in volts, d[k-1] be the

previous binary decision represented as a sign in {-1,+1}, and V_T be

the decision threshold in volts. The implemented correction is

followed by

b_hat[k] = 1 when y[k] ≥ V_T, and b_hat[k] = 0 when y[k] < V_T.

Here V_tap is `dfe_tap_v`, y[k] is the corrected sample, and b_hat[k]

is the binary decision. [VERIFIED FROM SOURCE CODE] The default threshold is

zero volts and the initial previous bit is validated to be either zero or

one.

## 7.2 Training and decision-directed operation

Nebula uses a hybrid mode. During the known training region, the expected bit

is used to update the previous-symbol feedback state. After `training_stop`,

the DFE uses its own decision, creating decision-directed feedback.

If b[k] is the expected bit, its sign is

s[k] = +1 when b[k] = 1, and s[k] = -1 when b[k] = 0.

The signed margin recorded by the implementation is

Positive m[k] means the corrected sample is on the expected side of the

threshold. [VERIFIED FROM SOURCE CODE] The DFE result records corrected

samples, decisions, expected bits, margins, error indices, error count, and

error-propagation events.

## 7.3 Phase selection

The receiver does not assume that the center of the nominal UI is optimal

after channel delay and CTLE filtering. `choose_sampling_phase()` evaluates a

grid of phases from 0 to one UI using the transient time step. For each phase,

it samples the waveform, applies the DFE while all phase-training feedback

symbols are known, and scores the phase using the tenth percentile of the

training-region signed margins:

Here φ is phase in seconds, T is the phase-training index

set, and S(φ) is in volts. [VERIFIED FROM SOURCE CODE] The selected

phase maximizes S(φ). Plateaus are resolved using the longest contiguous

best run and a reference phase derived from the channel delay.

[ENGINEERING INTERPRETATION] Using a lower-tail percentile rather than only a

mean margin makes phase selection sensitive to the weaker part of the

training distribution. It is still a finite-pattern heuristic, not an

eye-opening theorem or a clock-recovery implementation.

## 7.4 Eye height and eye width

For each candidate phase, the implementation separates corrected samples

belonging to logical ones and zeros. The vertical DFE eye height is computed

from the tenth percentile of the one samples and the ninetieth percentile of

the zero samples:

The best phase is the phase with the largest finite H_eye. A phase is

considered open when its height is at least the configured minimum height.

The implementation then finds the contiguous open region containing the

locked phase and converts its number of phase samples to unit intervals:

Here N_open is the contiguous sample count and Δ t_phase is the

phase-step duration in seconds. [VERIFIED FROM SOURCE CODE] At candidate

fidelity and above, the receiver requires locked-phase eye height above 0.1 V

and eye width above 0.4 UI. Those thresholds are implementation gates, not a

claim that they are a complete PCIe compliance specification.

## 7.5 Error count and decision margin

For the measured region M, the implementation records

and

The reported minimum decision margin is

[VERIFIED FROM SOURCE CODE] The transient stage rejects a candidate if the

post-warm-up error count is nonzero or if the minimum margin is not positive.

Zero observed errors means only that the finite simulated pattern contained

no observed decision errors under the tested condition. [ENGINEERING

INTERPRETATION] It is not a measured bit-error rate at an extrapolated target

confidence and does not prove long-run or silicon BER.

## 7.6 Behavioral-model assumptions and limitations

The model assumes a known deterministic stimulus, an idealized sampling

operation, a scalar one-tap correction, a fixed threshold, and a feedback

state that switches from known training bits to hard decisions. It does not

model transistor noise, clock jitter, sampler kickback, comparator offset,

metastability, finite feedback delay, adaptation dynamics, mismatch, or

physical DFE power and area.

[VERIFIED FROM SOURCE CODE] The generated eye artifact labels the CTLE trace

as a continuous transistor-level CTLE output and the DFE points as behavioral

one-tap corrected samples; it explicitly states that the DFE plot is not a

transistor-level DFE waveform. [REPORTED IN PPT] The presentation shows the

behavioral DFE boundary as a project limitation. A transistor-level DFE

implementation is not available in the provided evidence.

# 8. SPICE Benchmarks and Simulation Metrics

## 8.1 Netlist templating and model use

[VERIFIED FROM SOURCE CODE] Each bench is a parameterized ngspice template.

`simulator.ngspice.run_simulation()` renders placeholders for the model

library, process corner, temperature, CTLE block path, stimulus include, and

output file. The request carries typed parameters and expected output

artifacts. A temporary run directory isolates generated files, and the result

contract records status, measurements, runtime, errors, return code, failure

stage, failure code, artifacts, and provenance.

The shared CTLE bench parameters include supply voltage V_DD in volts,

input common-mode voltage V_CM in volts, output load capacitance C_LOAD

in farads, source resistance per leg in ohms, differential termination in

ohms, and input parasitic capacitance in farads. Exact defaults are read from

`SimulationConditions` and the bench templates; a value not present in the

relevant artifact is not inferred here.

## 8.2 DC operating-point benchmark

`ctle_dc.cir` performs an `.op` analysis and prints input, output, source,

tail, and supply-current quantities. [VERIFIED FROM SOURCE CODE] The receiver

uses these measurements to form DC metrics and checks output offset, branch

current balance, output common-mode behavior, rail headroom, and CTLE power.

The DC power relationship represented by the evaluator is

where I_VDD is the ngspice supply-current sign convention and the leading

minus sign converts the source current convention into consumed power.

[ENGINEERING INTERPRETATION] DC screening is placed first because a failed

bias point makes later AC and transient metrics difficult to interpret. The

DC result is also not total receiver power: the Stage-1 circuit contains an

ideal tail source and does not include a physical sampler or physical DFE.

## 8.3 AC transfer benchmark

`ctle_ac.cir` applies a differential small-signal excitation with opposite

AC polarity on the two inputs, sweeps logarithmically from 1 MHz to 20 GHz,

and writes transfer magnitude, phase, input magnitude, and output magnitude.

[VERIFIED FROM SOURCE CODE] The logical differential transfer is

and gain in decibels is

The parser interpolates gain at 1 MHz, 10 MHz, 100 MHz, 1.25 GHz, 2.5 GHz,

and 5 GHz. It identifies the peak in the intended 1.25–2.5 GHz band and

computes CTLE peaking relative to 100 MHz. It additionally checks whether an

out-of-band peak is excessive and whether the 2.5–5 GHz shelf is controlled.

The phase trace is unwrapped and converted to group delay using

The implementation records mean and span over 100 MHz to 5 GHz and emits a

warning if the span exceeds the documented implementation threshold.

## 8.4 CTLE transient diagnostic

`ctle_transient.cir` uses a channel-free terminated differential waveform to

check that the CTLE produces a nonzero output swing and receives the expected

terminated input amplitude. [VERIFIED FROM SOURCE CODE] The evaluator parses

`vin_diff` and `vout_diff`, verifies the input peak-to-peak excursion, and

rejects zero output swing.

This is a diagnostic rather than the final receiver metric. It isolates the

CTLE and its termination from channel filtering and DFE sampling, allowing a

failure to be assigned to the local circuit before a more expensive channel

transient is attempted.

## 8.5 Channel-driven transient benchmark

`receiver_transient.cir` writes the input differential voltage, output

differential voltage, both output nodes, and supply current. The evaluator

filters the generated NRZ waveform through the channel before writing the

stimulus include. It then aligns the waveform by integer bulk delay, selects

a fractional sampling phase, applies the behavioral DFE, and computes eye,

margin, error, delay, loss, and average transient-power metrics.

[VERIFIED FROM SOURCE CODE] The transient configuration changes by fidelity:

SCREENING uses 32 bits, TRAINING uses 128 bits, CANDIDATE uses 512 bits, and

FINAL uses 1024 bits. Warm-up and tail symbols are excluded from the measured

region. The measured bit count can therefore be lower than the requested

stimulus count after warm-up, tail, and channel-delay removal.

The average transient power is computed as

over the valid measurement interval. Output rail excursions are also checked

against the nominal supply and ground with the implementation's interpolation

tolerance.

## 8.6 Input-referred noise benchmark

`ctle_noise.cir` runs ngspice noise analysis with the differential output

observation and a differential test source. The output includes input-noise

spectral density and output-noise spectral density. [VERIFIED FROM SOURCE CODE]

`integrated_input_noise()` integrates the square of the input-referred

density over frequency:

where S_v,in(f) is input-referred noise density in V/√(Hz),

f_1 and f_2 are the analysis limits in hertz, and trapezoidal numerical

integration is used. The implemented bench spans 10 MHz to 5 GHz. The noise

stage rejects values at or above 1.5 mVrms.

[ENGINEERING INTERPRETATION] This is an integrated small-signal noise result

for the modeled CTLE and test source. It does not include unmodeled physical

DFE, clock, package, or post-layout noise contributions.

## 8.7 HD3 benchmark

`ctle_hd3.cir` applies a differential sinusoidal input and records the output

differential waveform. The bench uses a 100 MHz fundamental and a differential

peak-to-peak amplitude supplied by the evaluator. [VERIFIED FROM SOURCE CODE]

At candidate fidelity one amplitude is used; at final characterization the

implementation evaluates 0.05 V, 0.1 V, and 0.2 V differential peak-to-peak

amplitudes. The reported gate metric is the 0.1 V case.

After discarding the first 100 ns, `hd3_db()` fits the output to fundamental,

second-harmonic, third-harmonic, DC, and slow-drift basis functions. If A_1

and A_3 are the fitted fundamental and third-harmonic amplitudes in volts,

the reported distortion is

The implementation requires steady-state duration, sufficient Nyquist margin,

and a well-conditioned fit. The gate requires the 100 mVpp result to be below

-30 dB.

[ENGINEERING INTERPRETATION] HD3 measures one specified sinusoidal operating

condition. It is not a broadband linearity proof, and the documented bench

frequency is not a sweep across the CTLE peaking band. [REQUIRES FURTHER

VERIFICATION] A full-amplitude, full-band distortion characterization is not

available in the provided evidence.

## 8.8 Metric extraction and failure semantics

| Analysis | Raw output | Extracted metric | Failure examples | Evidence |
| --- | --- | --- | --- | --- |
| DC | Printed operating-point values | Bias, current, common-mode, power | Missing model, invalid bias, current imbalance | [VERIFIED FROM SOURCE CODE] |
| AC | `wrdata` transfer trace | Gain, phase, peaking, shelf, group delay | Missing trace, invalid magnitude, uncontrolled response | [VERIFIED FROM SOURCE CODE] |
| CTLE transient | `vin_diff`, `vout_diff` | Input/output swing | Small input or zero output | [VERIFIED FROM SOURCE CODE] |
| Channel | Touchstone matrix | Loss, delay, singular value, causality ratio | Wrong impedance, insufficient bandwidth, non-passive/noncausal screen | [VERIFIED FROM SOURCE CODE] |
| Receiver transient | Output nodes and supply current | Eye, width, margin, errors, average power | Error, nonpositive margin, rail excursion, insufficient eye | [VERIFIED FROM SOURCE CODE] |
| Noise | Input/output noise spectra | Integrated input-referred Vrms | Missing trace or threshold violation | [VERIFIED FROM SOURCE CODE] |
| HD3 | Differential transient | Third/fundamental ratio | Insufficient cycles, ill-conditioned fit, threshold violation | [VERIFIED FROM SOURCE CODE] |

## 8.9 Reproducible benchmark flow

The benchmark flow is deterministic at the software level when its parameter

vector, model library, simulator, channel, stimulus, fidelity, and conditions

are held constant. [VERIFIED FROM SOURCE CODE] The evaluator records checksums

for the CTLE block, all bench files, implementation modules, model-library

dependencies, channel, and simulator identity. Temporary waveform artifacts

are parsed into scalar metrics and normally discarded, except when an explicit

eye capture is requested.

The exact raw AC, noise, and HD3 traces for every historical result are not

all retained as standalone report-ready plots. [NOT AVAILABLE IN THE PROVIDED

EVIDENCE] Where only scalar result JSON is present, this report does not

invent a waveform plot.

# 9. Hierarchical Evaluation Pipeline

## 9.1 Evaluation contract

[VERIFIED FROM SOURCE CODE] `simulator.receiver.evaluate_receiver()` is the

controlling implementation for one candidate evaluation. Its inputs are a

`ReceiverParameters` object, `SimulationConditions`, an

`EvaluationFidelity`, a channel path and port map, SKY130 configuration,

ngspice configuration, and an optional evaluation cache or eye-capture object.

Its output is a `ReceiverEvaluation` containing the stage sequence, success

status, measured metrics, violations, warnings, failed stage, runtime,

evaluation identity, and provenance.

The evaluator is hierarchical because it increases computational and signal

fidelity only after earlier checks succeed. Conceptually, for stages

s_1,…,s_n:

E(θ, c) is the failure record at stage s_j, if s_j is the first failed

stage; otherwise E(θ, c) is the complete metric vector, if all required

stages succeed.

Here θ is the parameter vector and c is the operating condition.

[ENGINEERING INTERPRETATION] This ordering avoids spending transient, noise,

HD3, or PVT runtime on a candidate whose bias or AC response is already

invalid.

## 9.2 Fidelity levels

[VERIFIED FROM SOURCE CODE] `EvaluationFidelity` defines SCREENING, TRAINING,

CANDIDATE, and FINAL. The receiver uses these levels to select transient bit

counts and to enable later stages.

| Fidelity | Transient request | Channel/receiver role | Noise and HD3 role |
| --- | --- | --- | --- |
| SCREENING | 32 bits | Early local circuit screening | Not enabled by the receiver stage list |
| TRAINING | 128 bits | RL rollout evaluation and channel/receiver metrics | Not enabled by the receiver stage list |
| CANDIDATE | 512 bits | Candidate qualification metrics | Noise and one-amplitude HD3 enabled |
| FINAL | 1024 bits | Final nominal characterization | Noise and three-amplitude HD3 characterization enabled |

The requested bit count is not necessarily the measured bit count. Warm-up,

tail, and integer channel delay are excluded from the valid measurement

window. [VERIFIED FROM SOURCE CODE] The historical qualification document

reports 960 measured bits for a nominal 1024-bit request. This is a measured

artifact-specific result, not a universal count for every condition.

## 9.3 Stage order and stopping behavior

[VERIFIED FROM SOURCE CODE] The runner list is constructed in this order:

1. setup and parameter/condition/model validation;

2. DC operating point;

3. AC response;

4. channel-free CTLE transient;

5. channel diagnostics at TRAINING and above;

6. noise and HD3 at CANDIDATE and above;

7. channel-driven receiver transient and DFE metrics at TRAINING and above.

Each stage returns a `StageResult`. A failed stage stops the sequence and

sets `failed_stage`. A simulator failure, parser error, invalid model path,

channel preflight failure, or hard metric violation is represented as

structured data rather than as an invented metric value.

## 9.4 Setup and validation

`ReceiverParameters.validate()` checks the legacy passive/current variables,

DFE tap bounds, and v3 geometry bounds. `SimulationConditions.validate()`

checks process-corner type, temperature, supply, common-mode voltage, load,

termination, and nonnegative input parasitic capacitance. SKY130 model-library

resolution and ngspice identity are checked before the first bench runs.

[VERIFIED FROM SOURCE CODE] A setup failure returns a failed evaluation with a

specific failure code such as invalid parameter, missing model library,

missing ngspice executable, or channel error. This is important to PPO and to

reporting: the simulator failure is observable as a failure stage, not

silently treated as an electrical zero.

## 9.5 Nominal filtering and PVT selection

[VERIFIED FROM SOURCE CODE] The pipeline's nominal feasibility filter runs

before PVT qualification. Only nominally feasible candidates are sent into

the more expensive condition sweep. The PVT selector can rank candidates

using nominal validity and available PVT results, but it cannot promote a

candidate for conditions that were not evaluated.

[ENGINEERING INTERPRETATION] This separation is a practical compromise:

candidate generation needs a cheaper feedback loop, while final acceptance

needs broader condition coverage. It also creates a reporting obligation:

nominal PASS, candidate-fidelity PASS, and PVT-qualified are different

claims and must remain separately labelled.

## 9.6 Stage-result table

| Stage | Main source | Inputs | Outputs | Failure consequence |
| --- | --- | --- | --- | --- |
| Setup | `receiver.py`, `config.py`, `ngspice.py` | Parameters, conditions, model/tool paths | Validated run context | No simulation; structured setup failure |
| DC | `ctle_dc.cir` | Bias and model parameters | Operating point, power, headroom | Later stages skipped |
| AC | `ctle_ac.cir`, `waveform.py` | Differential AC source | Gain, peaking, shelf, group delay | Later stages skipped |
| CTLE transient | `ctle_transient.cir` | Short NRZ waveform | Input/output swing | Later receiver work skipped |
| Channel | `channel.py` | S4P and port map | Loss, delay, passivity, causality diagnostics | Receiver transient skipped |
| Noise | `ctle_noise.cir` | Small-signal noise setup | Integrated input-referred noise | Candidate rejected if threshold fails |
| HD3 | `ctle_hd3.cir` | Sinusoidal input | Third-harmonic ratio | Candidate rejected if threshold fails |
| Receiver transient | `receiver_transient.cir` | Channel-filtered waveform | Eye, DFE, errors, margin, power | Candidate rejected if hard gates fail |

Figure 2. Optimization, nominal filtering, PVT, final HD3/noise, and

reporting flow shown in the supplied presentation. Evidence:

`report_generation/figures/ppt_optimization_flow.png`, extracted from the

uploaded PPT; [REPORTED IN PPT]. The Python implementation is the controlling

source for the exact stage behavior.

## 9.7 Engineering contributions

[VERIFIED FROM SOURCE CODE] The engineering work represented in the repository

includes the following concrete components:

- a parameterized SKY130 CTLE subcircuit and reusable SPICE benches;

- Python netlist rendering, ngspice process control, waveform parsing, and

structured failure contracts;

- four-port S-parameter parsing, differential-transfer formation, channel

passivity/delay/causality screens, and waveform filtering;

- behavioral one-tap DFE sampling, phase selection, eye metrics, margins, and

finite-pattern error counting;

- PPO v1, the validity-aware and reward-structured v2 changes, and the

versioned eight-parameter v3 contract;

- Random Search and CEM implementations with result consolidation;

- content-addressed evaluation caching, dependency hashing, provenance,

checkpointing, resumable training, and event/run-graph records;

- strict PASS/FAIL/NOT CLAIMED reporting, schematic export, dashboard

integration, and the natural-language orchestration wrapper;

- unit, integration, synthetic-mechanics, artifact-integrity, and optional

real-ngspice tests.

[ENGINEERING INTERPRETATION] These contributions form an evaluation and

design-space exploration framework. They do not by themselves constitute a

fully physical receiver, a layout database, or an optimizer optimality proof.

# 10. Parameter Space and Design Constraints

## 10.1 Versioned parameter schema

[VERIFIED FROM SOURCE CODE] `simulator.design_schema.py` defines one ordered

legacy schema for v1/v2 and one expanded ordered schema for v3. The physical

parameter vector is mapped to discrete grids by `rl.parameter_grid.py`; the

optimizer therefore operates on grid indices rather than arbitrary floating

point values.

| Parameter | Meaning | Range/Value | Scaling | Engineering role |
| --- | --- | --- | --- | --- |
| `rload_ohm` | Drain-load resistance | 100 to 10,000 Ω | Logarithmic | Gain, bandwidth, output operating point |
| `rdeg_ohm` | Source-degeneration resistance | 10 to 10,000 Ω | Logarithmic | Local feedback, linearity, peaking |
| `cdeg_f` | Degeneration capacitance | 10 fF to 10 pF | Logarithmic | Frequency-dependent degeneration bypass |
| `itail_a` | Tail current | 10 µA to 1 mA | Logarithmic | Bias, transconductance, power |
| `dfe_tap_v` | Behavioral DFE tap | -0.4 to 0.4 V | Linear | Previous-symbol correction |
| `mos_width_um` | Matched transistor width | 0.42 to 100 µm | Linear | Drive, capacitance, partial area |
| `mos_length_um` | Matched transistor length | 0.15 to 1.0 µm | Linear | Device behavior and partial area |
| `mos_multiplier` | Matched parallel-device multiplier | Integer 1 to 16 | Discrete integer | Drive and partial area |

The first five rows are [VERIFIED FROM SOURCE CODE] legacy bounds. The last

three are v3 bounds. [VERIFIED FROM SOURCE CODE] Both MOS devices receive the

same v3 geometry values; independent unmatched widths or lengths are not

exposed by the schema.

## 10.2 Grid representation

The v1/v2/v3 environments use parameter grids with an ordered index for each

parameter. For a logarithmically scaled variable x with bounds x_min and

x_max, a normalized position can be represented as

For a linearly scaled variable,

The exact grid-point count and arrays are checkpoint/runtime-contract data.

[VERIFIED FROM DOCUMENTATION] The v3 default grid has 21 points for the

continuous variables, rounds MOS width and length to 0.001 µm, and uses the

integers 1 through 16 for the multiplier. [REQUIRES FURTHER VERIFICATION]

Any claim about a candidate outside the checkpoint's stored exact arrays

requires checking that checkpoint rather than assuming the default grid.

## 10.3 Action reachability

The default action is not a direct physical assignment. It selects an index

delta, which is clipped to the grid:

where i_j is the current index, N_j is the number of grid points, and

Δ_j ∈ {-1,0,+2} by default. [VERIFIED FROM SOURCE CODE]

`rl.autockt_action.apply_action()` uses three independent Discrete(3) choices,

one per parameter, and clips at the bounds. A boundary action can therefore

request movement without changing the applied index.

[ENGINEERING INTERPRETATION] The asymmetric `+2` step can reach farther in a

short horizon than a symmetric one-step action, but it also makes local

movement directionally uneven. The repository includes an optional symmetric

ablation using `(-1,0,+1)`; the default historical contract remains

`(-1,0,+2)`.

## 10.4 Constraint types

The parameter bounds are not the same as electrical acceptance criteria. The

former define what the optimizer may request; the latter are evaluated by

SPICE and receiver analysis. The distinction is:

| Constraint | Type | Example consequence |
| --- | --- | --- |
| Parameter bounds | Search-domain constraint | Invalid geometry is rejected before simulation |
| Grid membership | Representation constraint | Candidate must correspond to a stored index |
| DC/AC gates | Electrical validity constraint | Candidate stops before receiver transient |
| Eye/DFE gates | Receiver constraint | Candidate fails on errors, margin, height, or width |
| Area status | Reporting constraint | Partial area cannot qualify total-area target |
| PVT coverage | Qualification constraint | Unrun conditions cannot count as passes |

## 10.5 Parameter-space limitations

[VERIFIED FROM DOCUMENTATION] v3 adds grouped MOS sizing but does not add

physical sampler, physical DFE, passive geometry, or layout variables. The

search space is therefore a transistor-level CTLE parameter space embedded in

a behavioral receiver evaluation, not a complete transceiver layout search.

[NOT AVAILABLE IN THE PROVIDED EVIDENCE] A DRC-feasible geometry grid,

extracted parasitic model, passive density model, or layout-aware area

constraint is not supplied.

# 11. PPO v1: Baseline Formulation

## 11.1 PPO role in Nebula

Proximal Policy Optimization is used as a sequential candidate generator. At

each environment step, a policy maps the current state to independent action

choices, the environment applies grid-index deltas, the receiver simulator

evaluates the resulting circuit, and the reward is returned to the trainer.

The policy is not a circuit surrogate. [VERIFIED FROM SOURCE CODE] Every

training step passes through `ReceiverRLAdapter` and then the real evaluator

when the real backend is selected. The policy chooses which designs to try;

ngspice determines their measured metrics.

## 11.2 v1 state vector

[VERIFIED FROM SOURCE CODE] `rl.autockt_state.build_state()` constructs a

13-element v1 state from three concatenated components:

where e_t is the four-element achieved-versus-target error vector, r_t is

the four-element target-versus-global-reference vector, and i_t contains

the five parameter indices. The normalized error function is

with a small denominator guard in Nebula. Here a is achieved metric value,

g is the target value, and the result is dimensionless. For the smaller-is-

better power specification, the signed error is reversed so that larger

positive values consistently indicate better satisfaction.

The direct state-builder documentation retains raw index semantics, while the

environment applies a normalized-index adaptation before passing indices into

the state construction path. [VERIFIED FROM SOURCE CODE] This distinction is

part of the project's AutoCkt compatibility notes and must not be collapsed

into a claim that every direct caller sees the same representation.

## 11.3 v1 action space

[VERIFIED FROM SOURCE CODE] v1 has five independent `Discrete(3)` heads.

Choice 0, 1, or 2 selects the default index delta -1, 0, or +2. The complete

action is therefore

The grid update and clipping equation is the one in Section 10.3. The action

mechanics are discrete, but `indices_to_normalized_action()` converts the

resulting physical grid values into the continuous [-1,1] encoding expected

by the existing receiver adapter.

## 11.4 v1 reward

[VERIFIED FROM SOURCE CODE] `rl.autockt_reward.autockt_reward()` uses the

signed relative errors of the four target metrics. It sums only negative

unsatisfied contributions and returns a terminal bonus when the total reaches

the documented threshold:

r_t is defined piecewise:

- r_t = the sum of negative excess errors, Σ e_j for j with e_j < 0,

when that sum is less than -0.02;

- r_t = 10.0, otherwise for a successful evaluation;

- r_t = -1.0, if the evaluation fails.

The values -0.02, 10.0, and -1.0 are source-code constants. The last branch

is a Nebula adaptation for structured simulator failures, not a claim about

the original AutoCkt implementation.

## 11.5 PPO policy and value networks

[VERIFIED FROM SOURCE CODE] `rl.ppo_agent.py` defines a policy network with

two hidden layers of 64 units each and tanh activations, followed by one

three-logit categorical head per parameter. The value network has a separate

two-layer tanh body and a scalar output. The source labels the 64-unit,

two-layer shape as AutoCkt-replicated, while the remaining optimizer and

network details are explicitly labelled unverified from the original AutoCkt

source.

The policy produces categorical distributions

and the value network estimates

Here φ and ψ are policy and value parameters. The exact PPO

hyperparameters not established by AutoCkt source are not restated as if

they were original-project constants. [NOT AVAILABLE IN THE PROVIDED

EVIDENCE] The original AutoCkt source does not pin every value used by the

Nebula standalone implementation.

## 11.6 Advantage estimation and clipping

[VERIFIED FROM SOURCE CODE] `rl.trainer.py` collects transitions and

`rl.ppo_agent.py` computes generalized advantage estimates with separate

terminated and truncated semantics. A conventional form is

and

where r_t is reward, γ is discount, λ is the GAE parameter,

and d_t is one only when the episode truly terminates. For a horizon or

budget truncation, the implementation can bootstrap from the reached state's

value instead of treating the artificial boundary as a terminal zero.

The PPO policy-ratio objective uses the standard clipped form

where r_t(φ) = π_φ(a_t|s_t) / π_φ_old(a_t|s_t) and ε

is the clipping parameter. [VERIFIED FROM SOURCE CODE] The implementation

contains this PPO update structure; exact constants should be taken from the

source file for any numerical reproduction.

## 11.7 Episode termination and failure handling

[VERIFIED FROM SOURCE CODE] The environment returns `done=True` on terminal

reward success and a separate `truncated=True` flag for horizon or shared

evaluation-budget exhaustion. A failed SPICE stage produces a structured

failure and reward rather than an uncaught simulator exception in the normal

environment path.

[ENGINEERING INTERPRETATION] This distinction matters because PPO's value

target should not treat “ran out of allowed steps” as equivalent to “the

design is physically terminal.” It also prevents a shared training budget

from producing fabricated transitions after the final permitted simulation.

## 11.8 v1 trade-offs and evidence limits

The v1 contract is compact and retains compatibility with the historical

five-parameter workflow. Its weaknesses are also explicit: a failed early

stage can provide little metric information, unmeasured quantities are not

distinguished in the original state representation, and a fixed initial

state can limit the observed learning problem.

[VERIFIED FROM DOCUMENTATION] The fair no-warm-start PPO benchmark recorded

0/20 successes at the selected budget, with all episodes failing at DC. This

is evidence about that configuration, not proof that PPO cannot solve the

design problem under another horizon, target distribution, or state/reward

contract.

# 12. PPO v2: State and Reward Improvements

## 12.1 Motivation and compatibility policy

PPO v2 is an additive, opt-in improvement. [VERIFIED FROM DOCUMENTATION]

The project preserves v1 behavior when the new environment arguments are

omitted. This allows historical v1 checkpoints and result files to remain

interpretable while v2 studies the effect of improved simulator information.

The central diagnosis was a reward-information problem: if every sampled

episode fails before the four target metrics are genuinely measured, a single

failure reward does not tell the policy whether one candidate was closer than

another. A second diagnosis was the state-information problem: filling

unmeasured metrics with zero can make “not evaluated” resemble “measured zero.”

## 12.2 Corrected reset evaluation

[VERIFIED FROM SOURCE CODE] v2 adds `evaluate_on_reset`. When enabled, the

environment evaluates the actual starting circuit before returning the initial

state. The state therefore reflects measured starting behavior rather than an

artificial all-zero metric dictionary.

This option is intentionally separate from the one-time initial-index

randomization feature. The reset contract can preserve a fixed initial grid

index for compatibility, while still evaluating that actual point. A reset

evaluation consumes simulator budget, so it is not enabled silently for all

historical runs.

## 12.3 Validity-aware state schema

[VERIFIED FROM SOURCE CODE] `rl.autockt_state_v2.py` extends the v1 state with:

- one validity flag;

- two early-stage margin dimensions for `ctle_power_w` and `peaking_db`;

- a one-hot failure-stage vocabulary.

The documented dimension is

The validity flag is not a recomputed guess. It decodes the validity half of

the existing adapter observation. The failure-stage one-hot vector includes

success, unevaluated reset, setup, DC, AC, CTLE transient, channel, transient,

noise, HD3, internal, and the synthetic out-of-region label.

For a metric m, the validity component is

q_m = 1 when m was genuinely measured, and q_m = 0 when m was not

available.

[ENGINEERING INTERPRETATION] This lets the policy distinguish a measured

zero from an unavailable value and distinguish an early DC failure from a

late transient near-miss. It does not create a measurement that the

simulator never produced.

## 12.4 Structured reward v2

[VERIFIED FROM SOURCE CODE] `rl.autockt_reward_v2.reward_v2()` retains the v1

success contract but adds explicit reward components and available-metric

tracking. A failure with no reliable target metrics receives the structural

floor `NO_INFORMATION_FLOOR_V2 = -20.0`. The source explains that this is a

floor for missing information, not a tuned circuit-performance result.

The v2 reward separates two concepts:

1. `autockt_terminal_success`, which preserves the v1 tolerance-based

terminal criterion; and

2. `strict_target_pass`, which checks raw target boundaries independently.

For a valid near-miss, the reward can retain measured constraint margins. For

a strict pass, the source adds a bounded overshoot bonus with

`OVERSHOOT_BONUS_SCALE = 0.1` and `OVERSHOOT_BONUS_CAP = 2.0`.

[VERIFIED FROM SOURCE CODE] These coefficient values are source constants.

They are not presented as experimentally optimized hyperparameters.

## 12.5 Early-stage information

DC and AC stages can produce genuine power and peaking measurements even when

the full eye metrics are absent. v2 exposes those measurements as early-stage

margins only when they are present in the metrics map. It does not use a

fabricated zero for a transient-only metric after a DC failure.

This preserves the failure boundary:

[VERIFIED FROM SOURCE CODE] The module documentation explicitly ties this

behavior to the receiver's existing stage results and validity flags.

## 12.6 v1, v2, and v3 comparison

| Feature | PPO v1 | PPO v2 | PPO v3 | Evidence |
| --- | --- | --- | --- | --- |
| Optimized parameters | 5 | 5 | 8 | [VERIFIED FROM SOURCE CODE] |
| State dimension | 13 | 27 | 52 | [VERIFIED FROM SOURCE CODE] / documentation |
| Reset measurement | Optional historical behavior; default no evaluation | Opt-in actual reset evaluation | Versioned v3 contract | [VERIFIED FROM SOURCE CODE] |
| Validity mask | Not represented in v1 state | Explicit validity information | Per-metric validity plus v3 features | [VERIFIED FROM SOURCE CODE] |
| Failure-stage encoding | Not represented in v1 state | One-hot stage vocabulary | Retained and extended in v3 state | [VERIFIED FROM SOURCE CODE] |
| Reward | Flat failure reward and terminal bonus | Structured valid/failure reward | Strict receiver reward plus partial-area regularizer | [VERIFIED FROM SOURCE CODE] |
| Checkpoint contract | Historical bare state dictionary | Compatibility-preserving legacy form | Metadata-bearing versioned export | [VERIFIED FROM SOURCE CODE] |

The v3 column is included for continuity; its full geometry and runtime

contract are developed in Section 13. [REQUIRES FURTHER VERIFICATION] The

state dimensions are implementation contract values and should be checked

against the exact checkpoint metadata when loading an individual artifact.

## 12.7 Evidence and limitations

The v2 design directly addresses observed information loss and simulator

failure handling. It does not demonstrate, by itself, that real-circuit PPO

outperforms Random Search or CEM. A controlled ablation file exists and the

test suite verifies its mechanics, but broad statistical conclusions require

the recorded seed count, budget, target pool, and backend to be reported with

each result.

[NOT AVAILABLE IN THE PROVIDED EVIDENCE] A multi-seed, matched-budget real-

SPICE study proving a statistically reliable v2 improvement over v1 is not

available as a completed general claim.

# 13. PPO v3: Expanded Sizing and Runtime Contract

## 13.1 Motivation for v3

[VERIFIED FROM DOCUMENTATION] PPO v3 expands the optimization space from the

five legacy variables to eight variables by adding a matched MOS width,

matched MOS length, and integer MOS multiplier. This is a meaningful model

change, not a relabeling of the v1/v2 checkpoint. The v3 contract therefore

uses a new state/action/checkpoint schema and rejects incompatible legacy

policies.

[ENGINEERING INTERPRETATION] The expansion addresses a physical limitation of

the earlier search: passive degeneration, load, bias, and behavioral DFE

settings could be adjusted, but transistor geometry remained fixed. It also

increases the search difficulty because each geometry changes both electrical

behavior and partial area.

## 13.2 v3 state construction

[VERIFIED FROM SOURCE CODE] `rl/autockt_v3.py::build_state_v3()` combines the

following state groups:

- target error and target-reference vectors;

- eight normalized parameter indices;

- normalized values for the v3 metric vocabulary;

- per-metric availability flags;

- one-hot failure-stage identity;

- channel loss features at 1.25, 2.5, and 5 GHz and fitted bulk delay;

- a normalized partial MOS channel-area feature;

- an explicit false total-area-valid flag.

The documented v3 dimension is 52. The exact composition is checkpoint

contract data and is not silently inferred when loading a policy. The channel

features are calculated from the selected channel path and port map, so a

policy's state includes information about the channel under which it is being

evaluated.

## 13.3 v3 reward and termination

[VERIFIED FROM SOURCE CODE] `reward_v3()` requires successful evaluation,

valid target metrics, all target specifications satisfied, CTLE peaking in

the 3--12 dB band, and zero measured DFE decision errors. It adds a small

partial-MOS-area regularizer only when that metric is available. The area

term is not a feasibility gate.

The strict success condition can be represented as

where S_sim means required simulation stages completed, S_target is

the strict target assessment, S_peak is the peaking-band condition, and

N_err is the measured finite-pattern error count.

[ENGINEERING INTERPRETATION] Separating reward shaping from final area

qualification is important. A smaller partial MOS channel area may influence

ranking during learning, but the system must not convert it into a complete

0.05 mm² receiver-area claim while passive and physical DFE areas are absent.

## 13.4 Checkpoint compatibility

[VERIFIED FROM SOURCE CODE] v3 inference exports contain metadata describing

the state schema, reward schema, ordered parameters, grids, action deltas,

channel contract, and toolchain context. The runtime contract checks these

fields before inference. A bare v1/v2 state dictionary is rejected under v3

rather than being interpreted with the wrong input dimension or parameter

meaning.

| Checkpoint type | Metadata requirement | Loading status |
| --- | --- | --- |
| Legacy v1/v2 policy | Historical bare state dictionary | Valid only under compatible legacy loading path |
| v3 inference policy | Metadata-bearing policy export | Accepted by v3 contract when fields match |
| v3 full checkpoint | Policy, value, optimizer, RNG, counters, grids, toolchain | Resumable only under compatible runtime contract |

[VERIFIED FROM RAW SIMULATION DATA] `results/ppo_v3_tt_1000_policy.pt` is a

genuine metadata-bearing v3 policy associated with the real-backend summary

`results/v3_tt_1000_summary.json`. [VERIFIED FROM LOGS] Direct loading and

candidate generation succeeded under the v3 compatibility check.

## 13.5 Resumption and reproducibility

[VERIFIED FROM DOCUMENTATION] Full v3 checkpoints preserve policy and value

weights, optimizer state, Python/NumPy/Torch random state, target/reset RNG,

update and evaluation counters, training settings, exact grids, channel

checksum/map, and toolchain identity. The in-memory evaluation cache is not

serialized, so resuming can repeat simulator work.

The distinction between `--updates` and `--max-evaluations` is material:

updates represent additional learning iterations, while the total evaluation

budget includes work already performed before resume. [ENGINEERING

INTERPRETATION] This prevents a resumed run from accidentally receiving a

new unreported simulation budget merely because its update count is reset.

## 13.6 v3 evidence and limits

The real v3 artifact records 100 updates and 692 evaluations with seed 101.

That demonstrates an executed real-backend training workflow. It does not,

without a matched multi-seed study, demonstrate convergence, superiority, or

generalization.

[NOT AVAILABLE IN THE PROVIDED EVIDENCE] The literal original shell command

used to produce the historical v3 checkpoint is not recorded. A reconstruction

from embedded metadata is possible, but it must remain labelled as a

reconstruction rather than an original command transcript.

# 14. Random Search and CEM Baselines

## 14.1 Purpose of baseline comparison

Random Search and CEM provide non-PPO references for the same design space.

They are useful only when parameter order, bounds, evaluation fidelity,

target, budget, initialization, and success definition are comparable.

[VERIFIED FROM SOURCE CODE] The repository retains separate implementations

and result schemas for both baselines.

## 14.2 Random Search

[VERIFIED FROM SOURCE CODE] `experiments/receiver_search.py` samples each

normalized parameter independently from a uniform interval:

where u_j is the normalized coordinate for parameter j. The coordinate is

then decoded using the parameter's linear or logarithmic scaling. The method

does not learn from earlier candidates.

[ENGINEERING INTERPRETATION] Random Search has broad coverage and simple

reproducibility, but it spends evaluations without using measured gradients,

failure stage, or local response. In a sparse feasible region, success

probability depends strongly on the initial distribution and budget.

## 14.3 Cross-Entropy Method

[VERIFIED FROM SOURCE CODE] `experiments/train_cem.py` samples a population

from a factorized Gaussian in normalized coordinates. The documented defaults

include initial mean zero, initial standard deviation 0.15, population five,

and elite count two for the relevant CEM configuration. After scoring a

population, the elite mean and standard deviation are updated, with a

standard-deviation floor retained by the implementation. A single-elite

fallback shrinks the spread by the documented factor.

For normalized coordinate vector u ∈ R^d,

and for elite set E,

The equations describe the implemented family of update; exact population,

elite, and fallback values are controlled by the source and run configuration.

## 14.4 Comparison evidence

| Optimizer | Search method | Budget | Best result | Runtime | Evidence |
| --- | --- | --- | --- | --- | --- |
| Random Search | Independent uniform normalized samples | 20 candidates in fair trial | 1/20 success | Not available in the comparison table | [VERIFIED FROM LOGS] |
| CEM | Gaussian population and elite update | 20 candidates in fair trial | 0/20 success | Not available in the comparison table | [VERIFIED FROM LOGS] |
| PPO | Sequential discrete policy actions | 20 candidates in fair trial | 0/20 success | Not available in the comparison table | [VERIFIED FROM LOGS] |
| CEM, warm-started | CEM near verified design | 13/30 planned before incomplete stop | 8/13 success | Not available in the comparison table | [VERIFIED FROM LOGS] Not comparable to uniform Random Search. |

[VERIFIED FROM DOCUMENTATION] The fair head-to-head is one seed and carries a

statistical-power caveat. The warm-started CEM result is not a fair advantage

claim because initialization and completion differ.

## 14.5 Baseline limitations

[ENGINEERING INTERPRETATION] A 0/20 result can reflect reachability, short

horizon, early DC failure, target difficulty, or insufficient budget. It is

not enough to conclude an algorithm is intrinsically incapable. A stronger

comparison requires multiple independent seeds, identical ordered schemas,

matched evaluation budgets, raw metrics for every candidate, wall-clock

timing, and predeclared success criteria.

[NOT AVAILABLE IN THE PROVIDED EVIDENCE] A statistically powered, multi-seed,

real-SPICE comparison establishing a reliable ordering among PPO, Random

Search, and CEM is not present.

# 15. Caching, Provenance, Checkpoints, and Run Graphs

## 15.1 Content-addressed evaluation cache

[VERIFIED FROM SOURCE CODE] `simulator.cache.EvaluationCache` stores scalar

evaluation results under content-derived identifiers. The receiver identity

includes parameters, operating conditions, fidelity, implementation hashes,

bench hashes, channel identity, model-library dependencies, simulator identity,

and channel port map where relevant.

Conceptually,

where H is the stable fingerprint function and the I terms are content

identities. A cache hit is safe only when all those inputs match.

[VERIFIED FROM SOURCE CODE] Cache writes use an atomic lock-file protocol.

Malformed or obsolete cache entries are treated as misses rather than fatal

results. Eye-capture evaluations bypass the scalar cache because they require

additional bounded waveform data.

## 15.2 Provenance

`simulator.provenance.py` records Git identity, SHA-256 file hashes, model

dependency closure, ngspice executable/version, solver selection, and schema

versions. [VERIFIED FROM SOURCE CODE] The evaluator hashes all bench files and

the implementation modules that control receiver metrics. The purpose is to

prevent a result produced by one code/model/toolchain state from being

mistaken for a result produced by another.

## 15.3 Checkpoint contents

Checkpointing operates at two levels:

- inference-only policy exports for candidate generation;

- full training checkpoints for resume.

[VERIFIED FROM SOURCE CODE] Full checkpoints preserve optimizer and RNG state

as well as model weights and runtime contract information. v3 inference

requires metadata, whereas historical bare policy files remain part of the

legacy compatibility surface.

## 15.4 Run graphs and structured events

[VERIFIED FROM DOCUMENTATION] The UI and training path write per-step events

and execution graphs containing reset calls, actions, parameter changes,

stages, cache origins, failures, skipped stages, selection, verification, and

artifacts. JSON retains the complete graph; the visual overview is limited to

180 nodes.

[ENGINEERING INTERPRETATION] A run graph complements scalar metrics by

showing why a candidate reached or failed a stage. It is especially useful

for distinguishing a policy decision, cache reuse, simulator failure, and

qualification rejection.

## 15.5 Artifact integrity

[VERIFIED FROM SOURCE CODE] The Phase 2 artifact package includes a SHA-256

ledger and a test that compares every archived member with the recorded digest.

The repository also preserves crash logs, JSONL experiment records, plots,

schematics, and checkpoint files rather than replacing historical failures

with later successful runs.

# 16. Nominal Real-SPICE Results

## 16.1 Fresh measured result

[VERIFIED FROM RAW SIMULATION DATA] The fresh evidence file

`results/llm_wrapper_runs/smoke_evidence_demo.json` records the following

nominal result for the selected design and its stated TT/1.8 V/27 °C context.

| Metric | Value | Unit | Source | Verification Status | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Eye width | 0.73 | UI | `smoke_evidence_demo.json` | [VERIFIED FROM RAW SIMULATION DATA] | Exceeds the implemented candidate gate for that run. |
| Locked-phase eye height | 1.55 | V | Same | [VERIFIED FROM RAW SIMULATION DATA] | Vertical opening at selected phase. |
| Decision margin | 0.7294 | V | Same | [VERIFIED FROM RAW SIMULATION DATA] | Minimum signed sampled margin. |
| CTLE power | 1.206 | mW | Same | [VERIFIED FROM RAW SIMULATION DATA] | CTLE-level power, not total receiver power. |
| AC peaking | 3.599 | dB | Same | [VERIFIED FROM RAW SIMULATION DATA] | AC metric in the implemented peaking definition. |
| HD3 | -75.62 | dB | Same | [VERIFIED FROM RAW SIMULATION DATA] | 100 mVpp nominal HD3 result. |
| Input-referred noise | 0.372 | mVrms | Same | [VERIFIED FROM RAW SIMULATION DATA] | Integrated noise over the implemented band. |
| DFE errors | 0 | count | Same | [VERIFIED FROM RAW SIMULATION DATA] | Zero in the finite measured pattern. |
| Partial MOS channel area | 3.113×10^-6 | mm² | Same | [VERIFIED FROM RAW SIMULATION DATA] | Explicitly partial; not total-area evidence. |

## 16.2 Result interpretation

[ENGINEERING INTERPRETATION] The nominal result demonstrates that one

metadata-compatible v3 policy can generate a candidate that reaches the

receiver-level gates and can be refined with noise and HD3 measurements. It

does not show that every policy rollout succeeds, that the design is best, or

that the nominal point remains valid over untested corners.

[VERIFIED FROM LOGS] The fresh nominal-only pipeline runtime was 330.67 s and

the smoke-evidence run with additional HD3/noise and PVT-smoke work was

1056.52 s. These times are run-specific and should not be generalized to all

machines or all candidates.

## 16.3 Historical specification artifact

[VERIFIED FROM RAW SIMULATION DATA] `results/design_a_final_specification.json`

contains PASS rows for measurable eye, power, peaking, HD3, and noise metrics,

while the transistor channel-area row is marked NOT CLAIMED for the complete

area requirement. This artifact is authoritative for its design and run

context; it is not evidence that total receiver area was measured.

## 16.4 Nominal result boundaries

The result is conditioned on the selected model library, channel, source and

termination settings, stimulus, fidelity, target, and checkpoint. [REQUIRES

FURTHER VERIFICATION] A result copied without those fields is not fully

reproducible. No silicon or post-layout measurement is associated with the

nominal JSON file.

## 16.5 User-supplied wrapper execution screenshot

[REPORTED IN PPT] A user-supplied screenshot titled by the user as proof of

the working wrapper shows a completed real-backend v3 execution. The visible

console record reports exit code 0, runtime 412.90 s, checkpoint

`results/ppo_v3_tt_1000_policy.pt`, and the following selected parameters and

measured rows:

| Metric | Visible value | Unit | Visible status | Evidence interpretation |
| --- | --- | --- | --- | --- |
| `rload_ohm` | 2457.14 | ohm | Selected parameter | [REPORTED IN PPT] Screenshot value; repository artifact remains the primary source. |
| `rdeg_ohm` | 485.714 | ohm | Selected parameter | [REPORTED IN PPT] |
| `cdeg_f` | 4.85714e-13 | F | Selected parameter | [REPORTED IN PPT] |
| `itail_a` | 0.00067 | A | Selected parameter | [REPORTED IN PPT] |
| `dfe_tap_v` | 0.0571429 | V | Selected parameter | [REPORTED IN PPT] |
| `mos_width_um` | 10.378 | um | Selected parameter | [REPORTED IN PPT] |
| `mos_length_um` | 0.15 | um | Selected parameter | [REPORTED IN PPT] |
| `mos_multiplier` | 1 | integer | Selected parameter | [REPORTED IN PPT] |
| Eye width | 0.76 | UI | PASS | Visible wrapper result; differs from other run artifacts and is not merged with them. |
| Eye height | 1.545 | V | PASS | Visible wrapper result; DFE/receiver metric as labelled by the wrapper. |
| Margin | 0.7281 | V | PASS | Visible wrapper result. |
| Power | 0.001206 | W | PASS | Visible CTLE power row, not total receiver power. |
| Peaking | 3.599 | dB | PASS | Visible AC metric. |
| HD3 | n/a | — | NOT CLAIMED | Screenshot explicitly says not executed, not a pass. |
| Input-referred noise | n/a | — | NOT CLAIMED | Screenshot explicitly says not executed, not a pass. |
| PVT | n/a | — | NOT CLAIMED | Screenshot explicitly says not executed, not a pass. |

[Figure unavailable: source image not found in the available project evidence]

Figure 3. User-supplied wrapper console output showing a successful real-

backend v3 pipeline invocation, selected circuit parameters, measured nominal

PASS rows, and explicit NOT CLAIMED rows for unrequested HD3, noise, and PVT.

Evidence: user-supplied attachment “Pasted text #1”; the binary image was

not available as a local project asset, so no image is embedded above. This

screenshot's text transcript proves a completed wrapper invocation and its

displayed verdicts; it does not independently prove the underlying raw

waveform or model-library provenance.

[ENGINEERING INTERPRETATION] The screenshot is useful interface evidence

because it shows the wrapper returning a structured result rather than merely

printing a generic success message. Its values must remain a separate run

snapshot unless matched to a raw JSON artifact by checkpoint, target, channel,

conditions, and evaluation identity.

Figure 4. Dashboard final-parameter cards and measured-specification

table from a captured Nebula run — the one supplied image that shows genuine

application interface chrome (labelled parameter fields and a verdict table),

as distinct from the regenerated analysis charts in Section 19. Evidence:

`report_generation/figures/ppt_final_parameters_specification.png`, extracted

from the uploaded PPT; [REPORTED IN PPT]. The visible PASS/NOT CLAIMED rows

are interface output for that captured run and are not merged with the fresh

JSON result table above.

## Final Selected Candidate

The table below records the candidate visible in the supplied working-wrapper

execution screenshot. It is kept separate from the fresh JSON result because

the screenshot reports a different eye width and runtime and does not expose

all provenance fields.

| Field | Value | Unit/status | Evidence |
| --- | --- | --- | --- |
| `R_LOAD` | 2457.14 | ohm | [REPORTED IN PPT] User-supplied wrapper screenshot |
| `R_DEG` | 485.714 | ohm | [REPORTED IN PPT] |
| `C_DEG` | 4.85714e-13 | F | [REPORTED IN PPT] |
| `I_TAIL` | 0.00067 | A | [REPORTED IN PPT] |
| `V_DFE` | 0.0571429 | V | [REPORTED IN PPT] |
| `W_MOS` | 10.378 | um | [REPORTED IN PPT] |
| `L_MOS` | 0.15 | um | [REPORTED IN PPT] |
| `M_MOS` | 1 | integer | [REPORTED IN PPT] |
| Optimizer/checkpoint | PPO v3 / `results/ppo_v3_tt_1000_policy.pt` | — | [REPORTED IN PPT] |
| Random seed | Unavailable in the screenshot | — | [NOT AVAILABLE IN THE PROVIDED EVIDENCE] |
| Channel | Unavailable in the screenshot | — | [NOT AVAILABLE IN THE PROVIDED EVIDENCE] |
| Model library | Unavailable in the screenshot | — | [NOT AVAILABLE IN THE PROVIDED EVIDENCE] |
| Supply | Unavailable in the screenshot | — | [NOT AVAILABLE IN THE PROVIDED EVIDENCE] |
| Temperature | Unavailable in the screenshot | — | [NOT AVAILABLE IN THE PROVIDED EVIDENCE] |
| Simulation fidelity | Unavailable in the screenshot | — | [NOT AVAILABLE IN THE PROVIDED EVIDENCE] |
| Wrapper runtime | 412.90 | s | [REPORTED IN PPT] |
| Eye height | 1.545 | V | PASS shown; DFE/receiver metric |
| Eye width | 0.76 | UI | PASS shown; DFE/receiver metric |
| HD3 | Unavailable; NOT CLAIMED | — | [REPORTED IN PPT] Not executed |
| Input-referred noise | Unavailable; NOT CLAIMED | — | [REPORTED IN PPT] Not executed |
| Power | 0.001206 | W | PASS shown; CTLE-level power |
| Partial MOS channel area | 3.113e-06 | mm² | Explicitly partial area |
| PVT status | Unavailable; NOT CLAIMED | — | [REPORTED IN PPT] Not executed |

[ENGINEERING INTERPRETATION] This is a selected candidate snapshot, not a

universal final-design claim. Complete area, total receiver power, SF/FS PVT,

HD3, and noise cannot be inferred from the screenshot's nominal PASS rows.

# 17. PVT Qualification Results

## 17.1 Condition sets

[VERIFIED FROM SOURCE CODE] The full grid is

with corners TT, SS, FF, SF, FS; supplies 1.71, 1.80, and 1.89 V; and

temperatures 0, 27, 75, and 125 °C. The agreed qualification grid is

using TT, SS, and FF. The minimal historical grid contains 27 TT/SS/FF

conditions with the documented temperature subset.

## 17.2 Historical finalist result

[VERIFIED FROM DOCUMENTATION] The dated v3 qualification artifact reports:

| Design | Nominal status | Corner result | Status | Verification |
| --- | --- | --- | --- | --- |
| `pipeline_ep0` | PASS at nominal 512 and 1024-bit contexts | 36/36 | Qualified for the documented grid | [VERIFIED FROM RAW SIMULATION DATA] Historical |
| `pipeline_ep1` | PASS at nominal contexts | 18/36 | Rejected by PVT | [VERIFIED FROM RAW SIMULATION DATA] Historical |

The corner ranges recorded for `pipeline_ep0` include HD3 from -79.8428 to

-69.6987 dB, input-referred noise from 0.000328262 to 0.00048729 Vrms, eye

height from 1.27665 to 1.65185 V, eye width from 0.71 to 0.78 UI, CTLE power

from 0.0011457 to 0.0012663 W, and zero observed errors across the reported

conditions. [VERIFIED FROM RAW SIMULATION DATA] These values are historical

candidate-fidelity results, not a fresh final-fidelity rerun.

## 17.3 Qualification semantics

A PVT pass is a conjunction over the evaluated condition set:

where D is a design, C is the required condition set, and

S(D,c) is the strict stage and metric success for condition c. If a

condition is not run, it cannot contribute a PASS. If a run is interrupted or

times out, the condition remains incomplete or failed according to the result

contract.

[VERIFIED FROM DOCUMENTATION] The historical artifact records 36/36 on the

TT/SS/FF grid. [NOT AVAILABLE IN THE PROVIDED EVIDENCE] A completed full

60-condition qualification is not present.

## 17.4 Fresh final-fidelity status

[VERIFIED FROM LOGS] A recorded final-fidelity run completed and passed three

of 36 conditions before it was stopped. The measured pace

was approximately eight minutes per condition. This is evidence of runtime

cost and partial progress, not a complete final-fidelity qualification.

## 17.5 Why both 27 and 36 appear in the evidence

The 27-point and 36-point results are different condition sets, not duplicate

claims. [VERIFIED FROM RAW SIMULATION DATA] The preserved

`design_a_pvt_minimal27.jsonl` and `_rerun.jsonl` files each contain 27

conditions covering TT/SS/FF, 1.71/1.80/1.89 V, and temperatures 0, 27, and

125 °C. The 75 °C row is absent from that minimal set.

[VERIFIED FROM RAW SIMULATION DATA] The v3 qualification artifact contains 36

required conditions covering the same three process corners and supply values,

but temperatures 0, 27, 75, and 125 °C. The historical `pipeline_ep0` result

is therefore 36/36 on the four-temperature TT/SS/FF grid. The 27-condition

heatmaps describe the separate minimal historical grid and should not be

read as 36-point qualification or as five-corner 60-point qualification.

[VERIFIED FROM SOURCE CODE] The full defined grid adds SF and FS and contains

60 conditions. No artifact in the provided evidence establishes 60/60.

# 18. PPO Learning and Optimizer Comparison

## 18.1 Training evidence

[VERIFIED FROM RAW SIMULATION DATA] The v3 summary records a completed real-

backend training run with seed 101, 100 updates, and 692 evaluations. Its

best-reward and candidate information are preserved in the summary JSON.

[ENGINEERING INTERPRETATION] This proves that the training loop can execute

through real evaluation and save a usable v3 policy. It does not establish

that the policy converged globally or that learning caused the eventual

success rather than the initialization, target, action reachability, or

selection process.

## 18.2 Fair comparison

| Optimizer | Search method | Budget | Best result | Runtime | Evidence |
| --- | --- | --- | --- | --- | --- |
| PPO | Sequential discrete policy | 20 | 0/20 | Not recorded in comparison summary | [VERIFIED FROM LOGS] |
| Random Search | Uniform normalized sampling | 20 | 1/20 | Not recorded in comparison summary | [VERIFIED FROM LOGS] |
| CEM | Gaussian elite update | 20 | 0/20 | Not recorded in comparison summary | [VERIFIED FROM LOGS] |

[VERIFIED FROM DOCUMENTATION] The study used one seed and carries an explicit

low-statistical-power caveat. The correct conclusion is that PPO superiority

was not demonstrated under this setup, not that PPO is universally inferior.

## 18.3 Unseen-target and frozen-policy evidence

[VERIFIED FROM RAW SIMULATION DATA] The controlled unseen-target artifact

records regression described as one win, six ties, and three losses for the

trained policy comparison. The project therefore does not claim reliable

generalization to unseen targets.

[VERIFIED FROM DOCUMENTATION] The v3 midpoint comparison evaluates frozen

policies and excludes pretraining cost. It is evidence about a specific

frozen-policy experiment, not a multi-seed learning comparison.

## 18.4 Learning conclusion

The defensible claim is narrow: Nebula implements PPO training, preserves

versioned checkpoints, and has real-backend training and inference evidence.

The evidence does not support a stronger claim that PPO is the most efficient

optimizer, generalizes reliably, or removes the need for baseline search.

## What the Optimizer Experiments Establish

[VERIFIED FROM SOURCE CODE] PPO, Random Search, and CEM are integrated into

the same broader receiver-evaluation workflow. [VERIFIED FROM LOGS] The fair

head-to-head record is PPO 0/20, Random Search 1/20, and CEM 0/20 under one

seed and a matched small budget. A separate warm-started CEM result is not

comparable to the unbiased trial.

These experiments establish implementation and feasibility evidence: each

optimizer can consume the project parameter schema and submit candidates to

the evaluation pipeline. They do not establish PPO superiority, near-

optimality, or global optimality. The comparison is limited by seed count,

evaluation budget, incomplete raw-data availability for some historical runs,

initialization asymmetry in non-controlled trials, and statistical power.

[ENGINEERING INTERPRETATION] The correct use of the comparison is to test

whether the optimizer integrations work and whether feasible designs can be

found under stated budgets. A conclusive optimizer ranking would require

matched multi-seed real-SPICE experiments, identical initialization policy,

raw metrics for every request, wall-clock measurements, and a predeclared

statistical analysis.

# 19. Eye Diagrams, AC Response, HD3, and Noise

## 19.1 Eye artifacts

[VERIFIED FROM RAW SIMULATION DATA] The repository contains

`results/v3_tt_1000_eye.png`, SVG, and bounded eye-source JSON artifacts. The

source data distinguish a continuous transistor-level CTLE output eye from

behavioral DFE corrected samples. The DFE panel must not be captioned as a

transistor-level waveform.

The eye artifact is generated after selection when eye capture is requested;

training does not capture and render eyes for every PPO step. Missing waveform

data are reported as not produced rather than fabricated.

## 19.2 AC response

[VERIFIED FROM SOURCE CODE] AC data are produced by the logarithmic 1 MHz to

20 GHz bench and reduced to scalar gains, peaking, response class, shelf

metrics, and group-delay metrics. The historical tuning artifact reports

three target frequencies, with one of three measured points within ±5%.

| Target | Measured | Within ±5% | Evidence |
| --- | --- | --- | --- |
| 1.25 GHz | 1.320527183368927 GHz | No | [VERIFIED FROM RAW SIMULATION DATA] |
| 1.875 GHz | 1.908910958457706 GHz | Yes | [VERIFIED FROM RAW SIMULATION DATA] |
| 2.5 GHz | 2.295117021187582 GHz | No | [VERIFIED FROM RAW SIMULATION DATA] |

This is sampled nominal tuning evidence. [REQUIRES FURTHER VERIFICATION]

Continuous tuning compliance and receiver/noise/HD3 compliance at every

tuning setting are not demonstrated.

## 19.3 HD3 and noise evidence

| Metric | Value | Unit | Condition | Source | Verification |
| --- | --- | --- | --- | --- | --- |
| HD3 | -75.62 | dB | Nominal, 100 mVpp differential | `smoke_evidence_demo.json` | [VERIFIED FROM RAW SIMULATION DATA] |
| Input-referred noise | 0.372 | mVrms | Nominal, 10 MHz–5 GHz | Same | [VERIFIED FROM RAW SIMULATION DATA] |
| Historical HD3 range | -79.8428 to -69.6987 | dB | 36 candidate-fidelity conditions | v3 qualification artifact | [VERIFIED FROM RAW SIMULATION DATA] |
| Historical noise range | 0.328262 to 0.48729 | mVrms | 36 candidate-fidelity conditions | Same | [VERIFIED FROM RAW SIMULATION DATA] |

## 19.4 Figure status

The eye PNG/SVG is available. [NOT AVAILABLE IN THE PROVIDED EVIDENCE] A

retained raw AC trace suitable for a new report-specific gain curve is not

available for every result discussed. Dashboard screenshots of the rendered

eye, PVT page, and run graph are not available as browser screenshots in the

supplied evidence (see §20.3–§20.4). The supplied PPT/evidence package

does contain regenerated analysis charts covering AC response, PVT

heatmaps, temperature sensitivity, and metric/runtime evolution, built from

stored 27-corner and per-request result data; these are included in the

report-local `figures/` directory with captions and are labelled as

presentation/regenerated-artifact evidence rather than fresh reruns. Several

of the supplied filenames (for example the two containing the word

"dashboard") do not describe an application screenshot — each image was

opened and inspected directly before captioning, and captions below describe

what is actually visible, not what the filename implies.

Figure 5. Repository-backed receiver eye artifact separating the

transistor-level CTLE output eye from the behavioral one-tap DFE sampling eye.

Evidence: `results/v3_tt_1000_eye.png`; [VERIFIED FROM RAW SIMULATION

DATA]. The DFE panel contains corrected sample points, not a physical DFE

waveform.

Figure 6. Captured metric and evaluation-time plots (CTLE power, locked-

phase eye height, eye width, and observed request time) against successive

evaluation requests, including reset/PVT/cache-hit requests. Evidence:

`report_generation/figures/ppt_metric_and_runtime_plots.png`; [REPORTED IN PPT].

The source image's own caption states that missing measurements and synthetic

values are not SPICE evidence; the visible header strip indicates the image

was cropped from a larger produced-report/schematic view.

Figure 7. Regenerated 27-condition (TT/SS/FF x 1.71/1.80/1.89 V x 0/27/125

degC) performance heatmaps for peaking, eye height, eye width, DFE margin,

power, noise, and HD3, titled "Measured performance across the completed

512-bit, 27-corner run" in the source image. Evidence:

`report_generation/figures/ppt_dashboard_overview.png`; [REPORTED IN PPT].

Despite its filename, this image is a regenerated analysis chart, not an

application/dashboard screenshot — confirmed by direct inspection. It is

evidence of the 27-point grid only, not of SF/FS coverage or the full

60-condition grid, and not of a completed FINAL-fidelity rerun. Two further

supplied images (`ppt_ac_corner_sensitivity.png`, `ppt_ac_peak_sensitivity.png`)

regenerate the peaking/peak-frequency slice of this same 27-condition data

from separate snapshots and are consistent with the values shown here; they

are not reproduced as separate figures to avoid near-duplicate content, but

remain available in `report_generation/figures/` as supplementary evidence.

Figure 8. Regenerated temperature sensitivity of peaking, eye height, eye

width, DFE margin, noise, and HD3 across the TT/SS/FF corners and three

supply voltages, at 0/27/125 degC. Evidence:

`report_generation/figures/ppt_dashboard_detail.png`; [REPORTED IN PPT].

Despite its filename, this image is also a regenerated analysis chart, not an

application/dashboard screenshot. It is drawn from the same 27-condition

result set as Figure 7 and does not extend coverage to 75 degC, SF, or FS.

Figure 9. AC gain-versus-frequency curves for the TT/SS/FF corners at

three supply voltages and three temperatures, regenerated from the recorded

27-corner artifact, with the intended 1.25-2.5 GHz band and peak marked.

Evidence: `report_generation/figures/ppt_ac_response_27_corners.png`;

[REPORTED IN PPT]. This is regenerated historical-artifact evidence, not a

fresh rerun; a second, separately regenerated version of the same underlying

chart type is also present in `figures/` (`ppt_temperature_sensitivity.png`,

which — despite its filename — is confirmed by inspection to be an AC

response chart, not a temperature-sensitivity chart) and is not reproduced

here to avoid duplicating Figure 9.

# 20. Dashboard and LLM Wrapper

## 20.1 Dashboard architecture

[VERIFIED FROM SOURCE CODE] `experiments/web_ui.py` exposes a local interface

for selecting backend, checkpoint, RL version, channel, target, PVT options,

HD3/noise options, and output artifacts. It launches the pipeline as a

subprocess instead of importing the core simulator and policy code into the

web server.

The UI supports circuit generation and PPO checkpoint training. It exposes

run graphs, plots, JSON results, schematics, and eye artifacts when those

artifacts were produced by the underlying run.

## 20.2 Natural-language wrapper

[VERIFIED FROM SOURCE CODE] `nebula/llm_wrapper.py` parses a user request,

constructs the pipeline command, runs it as a subprocess, and returns the

structured output. Deterministic parsing is available through the local

parser; an optional Anthropic provider is also implemented with fallback

behavior.

The wrapper does not run a separate metric model. It does not calculate eye

height, invent a PVT verdict, or replace the deterministic final-specification

assessment. [ENGINEERING INTERPRETATION] This division keeps natural-language

convenience separate from electrical truth.

## 20.3 Interface evidence

| Interface feature | Evidence | Status |
| --- | --- | --- |
| Generate and verify circuits | `experiments/web_ui.py` and wrapper outputs | [VERIFIED FROM SOURCE CODE] |
| Train a v3 checkpoint | UI command construction and training path | [VERIFIED FROM DOCUMENTATION] |
| Download graphs and JSON | Results and graph artifacts | [VERIFIED FROM RAW SIMULATION DATA] |
| Final-parameter and measured-specification card view | Figure 4 (§16.5) | [REPORTED IN PPT] |
| Browser screenshot of the control panel (target/backend/channel/checkpoint selectors) | No such screenshot found in the supplied evidence | [NOT AVAILABLE IN THE PROVIDED EVIDENCE] |
| Browser screenshots for every screen (run graph, PVT screen, rendered eye) | No complete screenshot set | [NOT AVAILABLE IN THE PROVIDED EVIDENCE] |
| LLM-generated electrical verdict | Not used; deterministic pipeline remains authoritative | [VERIFIED FROM SOURCE CODE] |

## 20.4 What the supplied images establish

Every image supplied for this report was opened and inspected directly before

being described here. Two of the supplied filenames contain the word

"dashboard" (`ppt_dashboard_overview.png`, `ppt_dashboard_detail.png`), which

could be read as implying a captured application screenshot showing the

control panel described in `experiments/web_ui.py` (target selection, backend

selection, channel choice, checkpoint selection). Direct inspection shows

neither image is that: both are regenerated matplotlib analysis charts (the

27-condition PVT heatmap grid and the temperature-sensitivity chart reproduced

as Figures 7 and 8 in §19.4). No browser screenshot of the control panel

itself is present in the supplied evidence. [NOT AVAILABLE IN THE PROVIDED

EVIDENCE] This is recorded as missing evidence, not implied by a filename.

[REPORTED IN PPT] The one supplied image that does show genuine application

interface chrome is `ppt_final_parameters_specification.png` (Figure 4,

§16.5): labelled parameter fields and a measured-specification table with

PASS/NOT CLAIMED verdicts. It establishes that the interface can render a

final-parameter card and a verdict table for a captured run. It does not

establish that every other displayed chart in this report was generated by

that same run, that any browser view was captured during the wrapper

screenshot's 412.90 s execution, or that the LLM itself designed or validated

a transistor circuit. [VERIFIED FROM SOURCE CODE] The repository remains the

primary evidence that the wrapper shells out to

`experiments.run_autockt_pipeline` and that the deterministic pipeline

computes metrics and verdicts.

# 21. Verification and Test Coverage

## 21.1 Live regression status

[VERIFIED FROM LOGS] The live command `python -m pytest -q tests/` completed

with 632 passed, 2 skipped, and 27 subtests passed. Older documents report

419 and 582 tests because the suite grew; those historical counts are not used

as the current total.

## 21.2 Test categories

| Area | Representative tests | What is checked |
| --- | --- | --- |
| Circuit and parser | `test_ctle.py`, `test_parser.py`, `test_ngspice.py` | Parameter handling, parsing, simulator contracts |
| Receiver | `test_receiver_components.py`, `test_evaluate_pvt_grid.py` | Stage metrics and PVT control flow |
| PPO v1/v2 | `test_autockt_rl.py`, `test_rl_ppo_v2.py` | Legacy behavior, validity, reward, truncation |
| Checkpoints | `test_rl_checkpoint.py`, `test_v3_integration.py` | Save/load, compatibility, metadata |
| Synthetic learning | `test_synthetic_benchmark.py`, `test_rl_ppo_v2_ablation.py` | Mechanics without ngspice |
| Baselines | `test_train_cem.py`, `test_benchmark_report.py` | Search mechanics and result consolidation |
| PVT and qualification | `test_pvt_sweep.py`, `test_qualification_sequence.py` | Condition sets, stopping, qualification gates |
| Reporting/UI | `test_final_specification.py`, `test_v3_dashboard.py`, `test_web_ui.py` | Verdicts, plots, interface command construction |
| Artifact integrity | `test_artifact_integrity.py` | SHA-256 package membership and contents |

## 21.3 What tests do not prove

[ENGINEERING INTERPRETATION] Unit and integration tests prove contracts and

control flow, not physical validity over all parameter values. Synthetic PPO

tests prove learning mechanics on the controlled synthetic landscape, not

analog circuit optimization. The optional SKY130 integration test depends on

local model-library configuration and can be skipped.

[NOT AVAILABLE IN THE PROVIDED EVIDENCE] Tests do not substitute for silicon,

post-layout extraction, jitter tolerance, mismatch/Monte Carlo, or a complete

PCIe compliance test suite.

# 22. Limitations and Missing Evidence

## 22.1 Physical-model limitations

- The DFE is behavioral, not transistor-level. [VERIFIED FROM SOURCE CODE]

- The tail current source is ideal in the Stage-1 CTLE. [VERIFIED FROM SOURCE CODE]

- Passive devices are ideal SPICE R/C elements, not layout-extracted devices.

[VERIFIED FROM SOURCE CODE]

- Physical sampler, clocking, DFE power, and DFE area are absent. [VERIFIED

FROM DOCUMENTATION]

- Total circuit area and total receiver power are not available. [NOT

AVAILABLE IN THE PROVIDED EVIDENCE]

## 22.2 Validation limitations

- The public IEEE/IBM channel is not PCI-SIG compliance evidence. [VERIFIED

FROM DOCUMENTATION]

- SF and FS corners exist in the 60-point grid but are not part of the

demonstrated 36-point qualification. [VERIFIED FROM SOURCE CODE]

- The final-fidelity 36-condition rerun for the fresh design is incomplete.

[VERIFIED FROM LOGS]

- Zero finite-pattern errors are not a BER guarantee. [VERIFIED FROM

DOCUMENTATION]

- Continuous tuning, jitter, mismatch, and Monte Carlo signoff are not

demonstrated. [NOT AVAILABLE IN THE PROVIDED EVIDENCE]

## 22.3 Optimization limitations

- Fair PPO superiority is not demonstrated. [VERIFIED FROM LOGS]

- Multi-seed real-SPICE optimizer statistics are missing. [NOT AVAILABLE IN

THE PROVIDED EVIDENCE]

- Arbitrary-start reliable convergence is not demonstrated. [VERIFIED FROM

DOCUMENTATION]

- Surrogate-assisted and multi-actor PPO are not implemented. [VERIFIED FROM

DOCUMENTATION]

## 22.4 Evidence-management limitations

- Some historical training commands are not recorded verbatim. [NOT

AVAILABLE IN THE PROVIDED EVIDENCE]

- Hardware CPU/GPU details are not supplied for all historical runs. [NOT

AVAILABLE IN THE PROVIDED EVIDENCE]

- No browser screenshot of the dashboard control panel (target, backend,

channel, or checkpoint selection) is present; two supplied files whose

names suggest a dashboard capture are, by direct inspection, regenerated

analysis charts rather than application screenshots (§19.4, §20.4). [NOT

AVAILABLE IN THE PROVIDED EVIDENCE]

- The Phase 2 artifact count and later checksum-ledger count differ by one

file and were not independently reconciled. [REQUIRES FURTHER VERIFICATION]

# 23. Conclusions and Required Next Work

## 23.1 Failed Approaches and Lessons Learned

The repository preserves several negative or incomplete outcomes that are

useful engineering evidence rather than defects to hide.

| Observed failure or gap | Lesson | Required next experiment |
| --- | --- | --- |
| Fair PPO trial reached 0/20 while Random Search reached 1/20 | PPO's current action reachability, horizon, initialization, and state/reward contract do not prove an advantage | Matched multi-seed, budget-matched real-SPICE comparison |
| Many candidates failed at early DC/AC stages | Early rejection is necessary, but early failures can provide little learning signal | Measure failure-stage distributions and compare v1/v2/v3 reward/state contracts |
| Historical 36/36 result covers TT/SS/FF only | Passing the agreed grid is not five-corner qualification | Complete SF/FS conditions in the defined 60-point grid |
| Area is only matched-MOS channel area | Partial geometry cannot support total-area compliance | Add passive, sampler, DFE, routing, and layout-overhead models |
| DFE remains behavioral | Receiver eye evidence does not prove a physical DFE | Implement and simulate a transistor-level sampler/DFE |
| Only one of three tuning targets met the recorded ±5% criterion | Nominal sampled tuning is not continuous tuning compliance | Run receiver/noise/HD3 checks across the required frequency range |
| Long runs are expensive and crash logs are preserved | Runtime and native-tool stability affect reproducibility | Record machine/toolchain data and run controlled repeated evaluations |

[VERIFIED FROM DOCUMENTATION] These observations come from preserved result

files, audit documents, and source-level failure handling. [ENGINEERING

INTERPRETATION] Each next experiment is a proposed closure step, not a claim

that the current repository already performs it.

## 23.2 Conclusions

Nebula demonstrates a functioning, versioned analog-design automation flow:

target specification enters a PPO or baseline search, candidates are mapped

to a parameterized SKY130 CTLE, ngspice produces staged measurements, a

behavioral DFE produces receiver-level metrics, and a deterministic reporting

layer records PASS, FAIL, or NOT CLAIMED.

[VERIFIED FROM SOURCE CODE] The project has clear ownership boundaries among

optimization, simulation, analysis, and interfaces. [VERIFIED FROM RAW

SIMULATION DATA] Real-SPICE nominal and historical qualification artifacts

exist. [VERIFIED FROM LOGS] The current software regression suite passes.

The strongest defensible result is therefore an integrated Stage-1

demonstration, not a complete physical receiver product. One historical

candidate passed the agreed 36-condition TT/SS/FF grid at candidate fidelity,

while another finalist failed part of that grid. Fresh nominal evidence

shows successful eye, power, HD3, noise, and finite-pattern error metrics for

one selected run. Neither result establishes total chip area, physical DFE

operation, silicon performance, or PCIe compliance.

## 23.3 Required next work

The following work is required to strengthen the engineering claim:

1. Implement a physical sampler and transistor-level DFE, including power and

area accounting.

2. Replace ideal passive assumptions with documented PDK or extracted layout

models.

3. Complete a defensible total-area model and retain the `NOT CLAIMED` gate

until all blocks and layout overhead are represented.

4. Run matched, multi-seed, budget-matched PPO/Random Search/CEM comparisons

with wall-clock and raw metric records.

5. Complete the full 60-condition PVT grid, or explicitly redefine the

qualification requirement with external approval.

6. Add PRBS15, jitter, mismatch, Monte Carlo, and long-run error testing.

7. Establish continuous tuning evidence across the required frequency range.

8. Obtain approved PCIe channel and compliance-test evidence.

9. Record original training commands, machine information, and complete

dashboard screenshots for future reproducibility.

[VERIFIED FROM DOCUMENTATION] These are repository-documented deferred items.

They are proposals for completion, not claims that the current repository

already implements them.

# References

Internal repository documents cited in this report

- `docs/V3_QUALIFICATION_RESULTS_20260913.md` — historical 36-condition PVT qualification results for `pipeline_ep0`/`pipeline_ep1` (§16, §17).

- `docs/AGREED_QUALIFICATION.md` — agreed qualification scope and simulated-bit-count record (§16, Appendix E).

- `docs/FINAL_TECHNICAL_AUDIT.md` — second qualified/near-qualified design record and SIGSEGV crash-log references (§16, §21).

- `docs/RL_PPO_V2_STUDY.md` — PPO v2 root-cause and state/reward design notes (§12).

- `docs/V3_TRAINING_AND_RUN_GRAPHS.md` — v3 training/run-graph documentation and an older test-suite snapshot (§13, §21, Appendix F).

- `docs/PHASE7_COMPLETION_REPORT.md` — an older test-suite snapshot (§21).

- `docs/stage1-decisions.md` — deferred Stage-2 scope (§23).

- `artifacts/PHASE2_ARTIFACT_INDEX.md` — Phase 2 artifact package file count, used in the file-count discrepancy note (Appendix F).

- `results/ppo_v3_tt_1000_policy.pt`, `results/v3_tt_1000_summary.json` — genuine v3 checkpoint and training summary (§13, §18).

- `results/benchmark_report.json` — fair PPO/Random Search/CEM head-to-head comparison (§14, §18).

- `results/baseline_comparison_summary.json` — non-controlled Random Search/CEM comparison (§14).

- `results/v3_qualification_final_20260913.json` — historical PVT qualification artifact (§17).

- `results/llm_wrapper_runs/smoke_evidence_demo.json` — fresh real-SPICE nominal and smoke-evidence run (§16, §19).

Primary source modules cited throughout — see Appendix A for the full index; principal entry points are `experiments/run_autockt_pipeline.py`, `simulator/receiver.py::evaluate_receiver`, `simulator/config.py`, `simulator/design_schema.py`, `rl/autockt_v3.py`, `nebula/llm_wrapper.py`, and `analysis/final_specification.py`.

External standards and tools referenced for context

- SkyWater Foundry / Google open-source SKY130 process design kit — source of the `sky130_fd_pr__nfet_01v8` device model used in `circuits/blocks/ctle.spice` (§6). Cited as the device-model source only; this report makes no claim of PDK-vendor validation or sign-off.

- ngspice, the open-source circuit simulator invoked via `simulator/ngspice.py` (§8–§9, §15). The supplied evidence does not pin a specific ngspice release number.

- Touchstone (SNP) S-parameter file format — used for the four-port channel models parsed by `simulator/channel.py` (§5).

- IEEE 802.3 — source naming convention for the supplied `ieee802_ibm_20db_thru.s4p` reference channel (§5); its own metadata states it is not PCIe compliance evidence, and this report repeats that caveat rather than asserting protocol conformance.

- PCI Express (PCIe) Gen-2 signaling context (5 GT/s, 2.5 GHz Nyquist) — used only to justify the bit-rate default in `simulator/stimulus.py` (§2, §5). This report does not cite a specific PCI-SIG specification revision and makes no claim of electrical, protocol, or compliance-test conformance to it (§22).

# 24. Appendices

## Appendix A. Source and artifact index

| Evidence group | Primary paths |  |
| --- | --- | --- |
| CTLE netlist | `circuits/blocks/ctle.spice` |  |
| SPICE benches | `circuits/benches/` |  |
| Receiver evaluator | `simulator/receiver.py` |  |
| Channel parser | `simulator/channel.py` |  |
| Waveform metrics | `simulator/waveform.py` |  |
| DFE and eyes | `simulator/receiver_metrics.py` |  |
| Parameter schema | `simulator/design_schema.py`, `simulator/config.py` |  |
| PPO v1/v2/v3 | `rl/autockt_*.py`, `rl/ppo_agent.py`, `rl/trainer.py` |  |
| Pipeline | `experiments/run_autockt_pipeline.py` |  |
| Baselines | `experiments/receiver_search.py`, `experiments/train_cem.py` |  |
| Analysis/reporting | `analysis/` |  |
| Tests | `tests/` |  |
| Fresh results | `results/llm_wrapper_runs/`, `report_generation/verified_results.md` |  |
| Historical qualification | `results/v3_qualification_final_20260913.json`, `docs/V3_QUALIFICATION_RESULTS_20260913.md` |  |
| Report-local figures | `report_generation/figures/` | PPT-derived figures and repository eye artifact |

## Appendix B. Evidence-status vocabulary

| Label | Meaning |
| --- | --- |
| [VERIFIED FROM SOURCE CODE] | Direct implementation or test evidence |
| [VERIFIED FROM RAW SIMULATION DATA] | Value read from a result artifact |
| [VERIFIED FROM LOGS] | Value confirmed by command or run logs |
| [REPORTED IN PPT] | Presentation context not sufficient by itself |
| [VERIFIED FROM DOCUMENTATION] | Explicit project documentation evidence |
| [ENGINEERING INTERPRETATION] | Technical interpretation, not a raw measurement |
| [NOT AVAILABLE IN THE PROVIDED EVIDENCE] | No supplied supporting artifact |
| [REQUIRES FURTHER VERIFICATION] | Partial, historical, or unresolved evidence |

## Appendix C. Important numerical results

| Metric | Value | Unit | Source | Verification Status | Interpretation |
| --- | --- | --- | --- | --- | --- |
| Fresh eye height | 1.55 | V | `smoke_evidence_demo.json` | [VERIFIED FROM RAW SIMULATION DATA] | Nominal selected run |
| Fresh eye width | 0.73 | UI | Same | [VERIFIED FROM RAW SIMULATION DATA] | Nominal selected run |
| Fresh CTLE power | 1.206 | mW | Same | [VERIFIED FROM RAW SIMULATION DATA] | CTLE-level only |
| Fresh HD3 | -75.62 | dB | Same | [VERIFIED FROM RAW SIMULATION DATA] | 100 mVpp nominal test |
| Fresh noise | 0.372 | mVrms | Same | [VERIFIED FROM RAW SIMULATION DATA] | 10 MHz–5 GHz integration |
| Historical PVT | 36/36 | conditions | v3 qualification artifact | [VERIFIED FROM RAW SIMULATION DATA] | TT/SS/FF candidate fidelity |
| Fair PPO | 0/20 | successes | benchmark report | [VERIFIED FROM LOGS] | One-seed comparison |
| Live tests | 632 passed, 2 skipped | tests | pytest output | [VERIFIED FROM LOGS] | Current regression result |

## Appendix D. Figure register

Figures are numbered in document order. "Content verified by direct

inspection" means the image file was opened and viewed during this reporting

pass and its caption describes what is actually visible, independent of its

filename.

| Figure | Section | Content | Source file | Status |
| --- | --- | --- | --- | --- |
| 1 | 6 | CTLE schematic (matched MOS pair, loads, RC degeneration, tail source) | `figures/ppt_ctle_topology.png` | Supplied PPT asset; content verified by direct inspection |
| 2 | 9 | Optimization and verification flow (candidate search; selection and verification) | `figures/ppt_optimization_flow.png` | Supplied PPT asset; content verified by direct inspection |
| 3 | 16 | Working-wrapper console result (text transcript only; no image file) | User-supplied attachment "Pasted text #1" | Image not mounted as a local project asset; see unavailable-figure notice |
| 4 | 16 | Final circuit-parameter cards and measured-specification table (PASS/NOT CLAIMED) | `figures/ppt_final_parameters_specification.png` | Supplied PPT asset; the one image confirmed to show genuine application interface chrome |
| 5 | 19 | Receiver eye diagram (CTLE output eye and behavioral DFE sampling eye) | `results/v3_tt_1000_eye.png` | Repository-backed raw artifact |
| 6 | 19 | Metric and runtime plots vs. evaluation request index | `figures/ppt_metric_and_runtime_plots.png` | Supplied PPT asset; content verified by direct inspection |
| 7 | 19 | 27-condition PVT performance heatmaps (peaking, eye height, eye width, DFE margin, power, noise, HD3) | `figures/ppt_dashboard_overview.png` | Supplied asset; filename says "dashboard" but content verified by direct inspection to be a regenerated analysis chart, not an application screenshot |
| 8 | 19 | Temperature sensitivity (peaking, eye height, eye width, DFE margin, noise, HD3 vs. temperature) | `figures/ppt_dashboard_detail.png` | Supplied asset; filename says "dashboard" but content verified by direct inspection to be a regenerated analysis chart, not an application screenshot |
| 9 | 19 | AC gain vs. frequency across the 27-corner grid (TT/SS/FF) | `figures/ppt_ac_response_27_corners.png` | Supplied PPT asset; content verified by direct inspection |

Supplementary, non-figure-numbered images present in `report_generation/figures/`

but not separately reproduced as report figures (to avoid near-duplicate

content already covered by Figures 7 and 9): `ppt_ac_corner_sensitivity.png`

and `ppt_ac_peak_sensitivity.png` (peaking/peak-frequency slices of the same

27-condition data as Figure 7, from separate regeneration passes) and

`ppt_temperature_sensitivity.png` (a second, separately regenerated AC

response chart of the same type as Figure 9, despite its filename). All three

were opened and inspected directly; none contradicts the figures above.

## Appendix E. Reproduction checklist

1. Use the repository revision and compatible Python environment.

2. Configure `NGSPICE_EXECUTABLE` and `SKY130_MODEL_LIBRARY`.

3. Select the correct RL version and metadata-compatible checkpoint.

4. Record target values, channel path, port map, fidelity, PVT condition set,

seed, horizon, and evaluation budget.

5. Run the requested pipeline and retain JSON/JSONL outputs and provenance.

6. Treat synthetic backend results as software-mechanics evidence only.

7. Do not promote partial area, incomplete PVT, or finite-pattern zero errors

into stronger claims.

8. Compare any new result with the source-linked evidence tables before

updating a conclusion.

## Appendix F. Unavailable information register

The following remain explicitly unavailable in the provided evidence:

- complete receiver/chip area;

- total receiver power;

- transistor-level sampler and DFE results;

- post-layout extraction and DRC closure;

- silicon or commercial-SPICE cross-validation;

- full five-corner 60-condition completed qualification;

- multi-seed statistically powered optimizer superiority;

- guaranteed BER, jitter, mismatch, and Monte Carlo signoff;

- continuous tuning compliance;

- complete dashboard screenshot set;

- original v3 training command and all machine specifications.

These omissions are limitations of the evidence base, not values to be

estimated from unrelated artifacts.

## Appendix G. Evidence and Reproducibility Notes

The report distinguishes source-code contracts, raw result artifacts, command

logs, historical documentation, and presentation screenshots. Source code

defines implemented behavior; raw JSON/JSONL and waveform artifacts support

measured values; logs support execution and timing; documentation records

historical scope and limitations; screenshots support only what is visibly

shown. These evidence classes are not interchangeable.

Reproducible claims should identify the repository revision, netlists and model

library, channel and port map, checkpoint/schema, seed, fidelity, operating

condition, target, result artifact, and whether the evidence is fresh,

historical, or synthetic. `simulator/provenance.py`, the ngspice wrapper, and

v3 checkpoint metadata implement much of this identity tracking. Missing

fields remain unavailable rather than being reconstructed from screenshots.

[Figure unavailable: the complete combined CTLE schematic-and-parameter image

supplied in the final review message is not mounted as a local project asset.

The report includes the available CTLE schematic figure and the source-backed

parameter table separately in Section 6.]

The 27-point and 36-point PVT artifacts are intentionally reported as

different grids: 27 points use TT/SS/FF with 0/27/125 °C, while 36 points use

TT/SS/FF with 0/27/75/125 °C. The defined 60-point grid adds SF and FS. This

distinction is part of the evidence record, not a presentation preference.